# Altera’s Cyclone is All Spin

### Introduction

Last Fall, Altera announced the release of a new low cost FPGA product line called “Cyclone”. As usual, they announced the imminent death of the ASIC industry at the same time. I have done some math and discovered, again, that we are not dead.

### Cyclone

Based on the Altera website I was able to come up with the data in the charts. Note immediately that their pricing is based on huge volumes in 2003 and 2004, which shows a certain degree of optimism in the stability of wafer pricing as we pull through the industry slowdown and also indicates that they can’t meet this pricing now. I’ll compare with our current CX4K numbers, which I have no doubt will have some downward flexibility in 2004 – I just don’t know what that is so I’m not going to build it into price models.

Also, there are some technical issues that we need to compare – one of which is the number of available I/Os versus the package size. Their 100 TQFP, for example, has just 64 user I/Os. There is a lot of overhead in programming these beasts and they are so power hungry, even small ones, that they need a lot of power and grounds to supply the juice.

These devices are SRAM programmable and so they still need the external EEPROM – added cost and board area for any system using the parts. The following analysis does not include the cost of the Altera EPROM.

| Device | Usable ASIC Gates | RAM | Package | Price (1) | Price (2) |
| --- | --- | --- | --- | --- | --- |
| EP1C3 | 21K | 59904 | 100TQFP | $7.00 | $4.00 |
| EP1C6 | 43K | 92160 | 144TQFP | $17.00 | $8.95 |
| EP1C12 | 87K | 239616 | 240PQFP | $35.00 | $25.00 |
| EP1C20 | 144K | 294912 | 324FPGA | $60.00 | $40.00 |

Note:

- Pricing based on 50K unit volumes in the 2003 timeframe

- Pricing based on 250K unit volumes in 2004 timeframe

### Chip Express Competitive Products

Obviously it’s not likely that our product line combination of memory and gates matches theirs slice for slice and so here is a combined table that indicates the closest slices that would be likely to address the needs of the equivalent design, whether limited by memory or gates.

| Device | Usable ASIC Gates | Match | RAM | Package | Price (50K) | Price (250K) | Chip Express Price Advantage (50K) | NRE | Unit cross-over |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CX4041 | 40K | EP1C3 memory and Gate Limited. EP1C6 Gate Limited | 64K | 100TQFP | $2.32 | $2.06 | $4.68 | $40K | 8547 |
| CX4101 | 100K | EP1C6 Memory Limited. EP1C12 Gate Limited | 96K | 144TQFP | $4.14 | $3.78 | $12.86 | $40K | 3110 |
| CX4251 | 250K | EP1C12 and EP1C20 | 320K | 240PQFP | $10.34 | $8.70 | $24.66 | $40K | 1622 |
| CX4251 | 250K | EP1C12 and EP1C20 – | 320K | 324FBGA | $11.20 | $9.51 | $48.8 | $40K | 819 |

Clearly the Chip Express pricing is very advantageous across the product line. We can illustrate this with some simple mathematics to determine how many units must be purchased of ASIC to recover the NRE. Look at the graph below - most customers would never reach the 50K unit pricing because they would be crazy to buy 50K units! Even the smallest Cyclone, assuming a 2003 50K unit price, cannot expect to reach volumes greater than about 10K units lifetime. The largest device is limited to just over 800 units before the project would be more economically achieved using a Gate Array ASIC.

| Differences | Chip Express Price Advantage (50K) | Chip Express NRE | Units purchased to recover NRE |
| --- | --- | --- | --- |
| EP1C3 vs CX4041 | $4.68 | $40,000 | 8547 |
| EP1C6 vs CX4101 | $12.86 | $40,000 | 3110 |
| EP1C12 vs CX4251 | $24.66 | $40,000 | 1622 |
| EP1C20 vs CX4251 | $48.8 | $40,000 | 819 |

### Conclusion

If chips are very small, the silicon can be inefficiently implemented – or implemented for a designer’s convenience – with little cost penalty. As chips get bigger, the cost of the silicon begins to dominate the cost of the package and so logic efficiency is more important and FPGAs rapidly become a costly solution.

Once again, the death of the ASIC is somewhat exaggerated. The Cyclone is just new spin on an old story.

| Device | Usable ASIC Gates | Match | RAM | Package | Price (50K) | Price (250K) | Chip Express Price Advantage (50K) | NRE | Unit cross-over |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## CX4041 | 40K | EP1C3 memory and Gate Limited. EP1C6 Gate Limited | 64K | 100TQFP | $2.32 | $2.06 | $4.68 | $40K | 8547 |
| CX4101 | 100K | EP1C6 Memory Limited. EP1C12 Gate Limited | 96K | 144TQFP | $4.14 | $3.78 | $12.86 | $40K | 3110 |
| CX4251 | 250K | EP1C12 and EP1C20 | 320K | 240PQFP | $10.34 | $8.70 | $24.66 | $40K | 1622 |
| CX4251 | 250K | EP1C12 and EP1C20 – | 320K | 324FBGA | $11.20 | $9.51 | $48.8 | $40K | 819 |

Estimated based on 12 ASIC gates per Logic Element and 60% array utilization