Have Your Cake And Eat It Too:

How To Optimize For Area AND Timing

Robert B. Wiegand

NxtWave Communications

rwiegand@nxtwavecomm.com

ABSTRACT

Timing closure and area optimization are often perceived as two mutually exclusive goals.  In reality, area optimization can have many positive effects on a design, including timing improvement.  Although there is no single magic switch to produce the smallest design that still meets timing, a combination of many factors can have a positive and dramatic impact on area in timing critical designs.  This paper will discuss the benefits of area reduction, address some concerns, and detail a whole “bag of tricks” that combine to significantly reduce area while maintaining timing.

- Introduction

With increasing clock frequencies and design complexities, as well as tighter time to market demands, area optimization can become the odd man out in ASIC design.  It’s the old “good, fast, cheap - pick any two”, where “good” represents timing/design closure, “fast” represents time to market, and “cheap” represents area and power optimization.  With time to market as a stake in the ground, area optimization and timing closure become perceived as mutually exclusive goals.  A striking example of this perception occurred to me at SNUG San Jose 2000 in a Synopsys tutorial titled “Getting the Most from Design Compiler for Area & Delay”.  The tutorial was presented in two distinct sections, one for area and one for timing.

Area reduction techniques are often dismissed because of such disclaimers as “not for use with timing critical designs.”  All designs are timing critical!  Another reason to dismiss area reduction techniques is that the design is schedule critical.  All designs are schedule critical!  With an understanding of what can be gained, it would seem that the best approach would be to build in area reduction techniques at the appropriate stages in the flow to gain the most benefit while minimizing any impact to timing and schedule.

- Why Size Matters

A gate saved is a penny earned.  In core limited designs, any die size reduction directly decreases the part cost by allowing more die per wafer. This, in turn, increases cost competitiveness and/or profit margin.  As production volumes increase, so does the cost benefit.  Reducing the placeable instance count and the number of wires each contribute to decreased routing congestion.  Producing a more routable design benefits both core and pad limited designs by improving time to market.  With fewer wires and decreased levels of logic, timing closure can be improved.  It is important to note, however, that reducing the die size to take advantage of reduced area will once again increase utilization and congestion (as well as edge capacitance effects), which effects timing closure.  One may “break even” on the timing closure aspect, but still maintain the cost benefits of area reduction.  Clearly there is a tradeoff.

Another area reduction benefit is improved power consumption.  Simply put, less gates use less power.  Driving less capacitance in fewer wires also uses less power.  Improved power consumption can influence package selection and marketability, both of which contribute to your company’s bottom line.  Finally, area reduction can improve reliability, testability, and yield.  Fewer gates and wires means fewer places for manufacturing defects to occur, keeping yields and margins up – another financial benefit.  Fewer gates and wires also mean fewer nodes to test for “stuck at” conditions, which can reduce tester time, yet another cost savings.  The bottom line is your bottom line.

- Addressing The Concerns

Assuming all designs are timing and schedule critical, the obvious concerns are how does one consider area optimization without adversely impacting timing closure and schedule.  Perhaps the single most effective safeguard is to have a standardized set of synthesis scripts and methodology with area reduction techniques built in.  This way, there is no script architecting (for area or anything else) as part of the project.  Not having to architect scripts from scratch for each project significantly reduces risk and allows time to consider area reduction techniques in the development of the flow.  With area reduction techniques “under the hood,” schedule pressure cannot be a reason to skip area optimization.  Area reduction techniques should be run on test cases independent of a project to verify their effectiveness and impact on timing.  Scripting for the project should consist of only exception management and mapping the design to the flow.  This allows flow development and optimization independent of the project with greatly reduced risk.  If exceptions are scripted or modified for area purposes, running test cases will minimize any perception of adding risk.

- Key Methodology Issues

Some of the major enemies of area optimization include schedule pressure, perceived risk, top down compile, over-constraining, failure to use selective ungrouping – especially on designware components, and failure to code for resource sharing in the RTL.  All of these issues can be managed by adhering to an established methodology.  As previously discussed, simply having a scripted synthesis methodology in place will address the first two issues.  The third, fourth, and fifth issues must be addressed within the scripts and methodology.  The final issue must be addressed before synthesis by including area considerations within an accepted and followed set of RTL coding guidelines.

  - Do Not Use Top Down Compile

One of the key methodology issues mentioned above concerns the use of top down vs. bottom up compile strategies.  Although top down compile strategies allow synthesis to have full visibility with respect to timing, it is very difficult to optimize for area with this approach.  Attempting to add an aggressive area constraint when performing a top down compile on a large design is simply not practical due to the impact on run time.  A far more practical and area friendly approach is to use a bottom up approach with aggressive max_area constraints.  To overcome the timing difficulties usually associated with such an approach, a multi-pass  bottom up approach which successively refines the lower level constraints (with characterize or budgeting) combined with top down incremental optimization should be used.  By using a divide and conquer approach, the bottom up strategy allows area optimization techniques like max_area to have greater impact with reasonable runtimes.  Although there are many viewpoints on what values to use for the max_area constraint, I have found that max_area 0 has produced consistently positive results when using a multi-pass approach.

  - Do Not Over-Constrain

Another key methodology issue involves the practice of over-constraining a design.  Over-constraining consumes area by causing designs to be overbuilt, using higher than necessary drive strengths on cells and forcing more area hungry implementations of designware components. Realistic and successively refined constraints are much more area friendly.  Default constraints, which should be built into the scripts and methodology, should reflect reasonable input & output delays, input drivers and output loading.  For example, use the “Q” output pin of a 1X drive flop as the default driving cell pin, and 4 times the “A” input pin of a 1X drive NAND gate as the default output load for a block.  For a default input delay, allow for the clock-to-Q of the driving flop plus a small wire delay.  For a default output delay, allow for setup time of the driving flop plus a small wire delay.  Use virtual clocks to constrain I/O’s of combinatorial blocks, or preferably ungroup combinatorial blocks into their parent designs.  Combinatorial and “snaking” paths, and possibly some sequential paths, will be under-constrained at first.  This would cause great difficulty in a single pass methodology.  By using a multi-pass methodology however, the constraints will be refined in subsequent passes without over-constraining the entire design.

  - Margin With Design Rules

Traditionally, over-constraining was done to minimize the timing losses incurred by non-timing savvy layout tools.  With the present widespread use of timing driven layout tools, timing accuracy (especially with respect to predicted clock insertion delay and skew) can be much more important than timing margin.  It is advisable, however, to margin the design with design rules instead of timing constraints.  One example is to tighten the max_transition value before layout, and relax it to an acceptable value after layout.  Another example is to use a max_fanout of 10 to 20 on compiled designs to reduce unexpected timing swings in layout.  Another example is to set a max_fanout on all the input ports of compiled blocks.  The max_fanout constraints on inputs and designs combine to prevent high fanout loading surprises further up the design hierarchy, and limit the amount of buffering to be performed by the timing driven layout tools.  I have found an input max_fanout value of one to be quite effective, and that this constraint can sometimes be lifted on the last pass in a multi-pass compile strategy.  Any resulting unnecessary buffering will be optimized away with the core level incremental optimization in synthesis.  Once again, these design rule based margining techniques should be built into the scripts and methodology.

  - Use Selective Ungrouping

Yet another key methodology issue involves the use of selective ungrouping and set_dont_touch on compiled designs.  Very small designs, especially purely combinational ones, should be ungrouped into their parent blocks whenever possible.  Compile generated hierarchy including designware and MUX OP components should also be ungrouped.  Ungrouping these components removes boundary conditions allowing further area optimization.  Ungrouping before the compile can result in longer run times, since synthesis has to look at more of the design at once.  Compiling hierarchically can have a runtime advantage, but consumes more area and also requires hierarchical saves, leading to multiple versions of the same leaf block throughout the design.  To obtain the best of both worlds, load the parent and leaf blocks and then compile hierarchically for the runtime advantage.  After the compile, ungroup the leaf blocks and compile generated hierarchy, apply a set_dont_touch on the design and save without the hierarchy switch. A subsequent high effort max_area 0 incremental compile will yield the area benefit of removing the hierarchical boundaries.

Do not perform compiles on purely hierarchical blocks containing no logic.  Instead, load the design, link, apply constraints including set_dont_touch, save (without the hierarchy switch) and run reports.  When a high level block contains mostly hierarchy but just a very small amount of logic, follow the above procedure with one variation.  After applying constraints, group the logic into a temporary design, compile the temporary design, and then ungroup the compiled logic back into the parent design before saving and running reports. This approach greatly improves run time, and saves area by preventing synthesis from inserting bogus buffering at higher levels in a design. Using a multi-pass methodology, violations in leaf blocks are fixed during subsequent compile passes.

  - RTL Coding Issues

Finally, area results can be further improved by including some area and resource sharing considerations within a set of RTL coding guidelines.  These considerations include inferring operators instead of instantiating designware components, writing sharable operators into the same always block, and using case statements with the infer_mux directive to infer register sourced MUXes.

By hand instantiating designware components, several area benefits are lost.  First of all, resource sharing will only occur when sharable operators are inferred within the same always block.  Resource sharing will not occur even when operators are inferred in separate wire statements.  By directly instantiating designware components, resource sharing is prevented.  Secondly, timing and area transformations like transform_csa and optimize_registers (also referred to as BOA/BRT, or Behavioral Optimization of Arithmetic and Behavioral ReTiming) cannot be performed on instantiated designware (optimize registers can, but not quite as optimally).  The transform_csa command can yield great timing and area benefits by merging multiple arithmetic operators into a single carry save adder (CSA) tree, saving the final (and only) carry propagate add for the end.  If a CSA tree is pipelined, the optimize_registers command can improve area by sliding a bank of registers down the tree to a stage requiring fewer registers if timing slack permits.  If not, timing is improved by sliding the registers to balance slack between the stages.  The transform_csa command requires inferenced operators, and both commands require a DC Ultra license.

Some traditional reasons for hand instantiating designware components are to use the carry in and carry out ports on an adder or to generate signed multipliers from Verilog RTL.  The carry in and carry out connections can be inferred as follows:

always @(a or b or carry_in) begin

{carry_out,sum}	= a + b + carry_in;

end

with the following variable setting:

hdlin_use_cin = true

Signed multipliers in Verilog are a bit more cumbersome, but can be inferred with a function call as shown in example 1:

module sgn_mult (a, b, y);

input [7:0] a, b;

output [15:0] y;

reg [15:0] y;

function [15:0] mult_tc; /*2's comp signed mult*/

input [7:0] A;

input [7:0] B;

reg sgn;

// synopsys map_to_operator MULT_TC_OP

// synopsys return_port_name Z

begin

// synopsys translate_off

sgn = A[7] ^ B[7];

if (A[7] == 1'b1) A = ~A + 1'b1;

if (B[7] == 1'b1) B = ~B + 1'b1;

mult_tc = A * B;

if (sgn == 1'b1) mult_tc = ~mult_tc + 1'b1;

// synopsys translate_on

end

endfunction

always @ (a or b) begin

y = mult_tc (a, b);	// a*b

end

endmodule

Example 1.  Coding a Signed Multipler in Verilog

It is also important to know when and when not to infer a MUX.  When the MUX’ed sources in a data path include constants, MUXes should not be inferred because they will prevent the logic reduction allowed by the constants.  When the MUX’ed sources are all register driven, MUXes will yield the best area benefit by reducing the gate count, number of placeable instances and the interconnect between them, as well as produce a more regular structure.  It is very important to know that the Verilog conditional operator construct C=SEL?(A:B) will not automatically infer a MUX in synthesis.  To guarantee a MUX, even a 2 to 1 MUX, use a completely specified case statement with the //synopsys infer_mux directive.  Without the infer_mux directive, even a completely specified case statement may still not produce a MUX in synthesis.  In addition, use the following variable setting:

hdlin_infer_mux = default

Setting this variable to “all” would eliminate the need for the infer_mux directive, but case statements with constants will also produce MUXes and lose the logic reduction allowed by the constants.

- MIN/MAX Methodology Revisited

Achieving significant area improvements over traditional synthesis approaches requires a methodology that addresses the key issues listed previously.  One such methodology is described in great detail in the SNUG99 San Jose paper “Using MIN/MAX Compile In A Multi-Pass Synthesis Flow.”  This methodology uses characterize to refine constraints between three bottom up compile passes.  A somewhat similar multi-pass approach is implemented in DC2000’s ACS with time budgeting instead of characterize using acs_compile_design, acs_recompile_design, and acs_refine_design.

As described in the MIN/MAX paper, lower level modules are compiled in a bottom up fashion using reasonable default constraints and design rule margining, as well as ungrouping of compile generated hierarchy.  At the core level, accurate and detailed constraints and exceptions are applied, and characterized to all the lower level modules.  Next, the lower level modules are compiled bottom up from the RTL once again, using the write script constraints from the top down parallel characterize.  Another characterize is performed from the core on down, followed by a third pass bottom up incremental compile.  Next, scan flops are stitched at the core level with insert_scan, followed by a core level top down incremental compile using min/max conditions to fix setup, hold and design rule violations.  These procedures can be applied to multiple cores.  Next, the clocks and pads blocks are compiled, followed by full chip netlist assembly, top level scan stitching, and final pre-layout optimization once again using min/max conditions to fix any remaining setup, hold, and design rule violations.  Finally, post-layout optimization is performed if necessary, once again using min/max conditions.  The flow is summarized in figure 1.

Figure 1.  Original MIN/MAX Flow

  - Using The Ping Pong Effect

As explained in great detail in the MIN/MAX paper, this methodology uses the ping-pong effect of characterize to refine constraints on each pass and close in on core level timing.  Given that parallel characterizing will assign the total path slack, positive or negative, to each module in that path, consider the paths through the two modules shown in Figure 2 as a simple example.

Figure 2.  Using the Ping Pong Effect of Characterize

After the first pass compile, path 1 has 2ns positive slack.  Following with a parallel characterize and a second RTL compile pass, path 1 now shows 2ns negative slack.  The relaxed constraints have allowed slower designware implementations, saving area at the expense of timing.  With one more parallel characterize followed by an incremental compile pass, we regain our positive slack (assuming our constraints are exactly met in this example).  Since the designware was ungrouped in the previous pass before the incremental compile, incremental implementation selection (IIS) was disabled, allowing the smaller implementations remain.

Path 2 starts off with 2ns negative slack after the first pass compile.  After a parallel characterize and a second RTL compile pass, path 2 now shows 2ns positive slack.  The tighter constraints have resulted in faster designware implementations, consuming more area to improve timing.  With a second parallel characterize followed by the third pass incremental compile, we might expect a return to negative slack.  Instead, the timing improvements remain because of the ungrouped faster designware implementations from the second pass.  In this case, area has been sacrificed for timing, but this was necessary since timing was not initially met.

  - Recent Improvements

While most of this methodology remains intact, recent tool improvements have allowed some changes and improvements to be implemented.  Originally, the pre-layout optimization was performed using compile –in_place with a whole battery of “compile_” variables set.  This step is now performed using compile –top.  The MIN/MAX over-pessimism problem has been quietly fixed in Design Compiler (as of 1999.05, I believe, but certainly in 1999.10-6), eliminating the need for the hidden variable workaround and all of the belt and suspenders multi-operating condition timing reports.  The MIN/MAX paper describes post-layout optimization in great detail, with and without the use of Floorplan Manager.  With the use of timing driven layout tools in a COT environment, the need for this step is eliminated, although Floorplan Manager (now part of DC Ultra) can still be used for custom wireload generation.  In its place, however, is the accurate conversion of Design Compiler or PrimeTime constraints for timing driven place and route.  The need for highly accurate and detailed full chip constraints is now even more critical, since timing driven placement can do just as much if not more damage than synthesis when given bad constraints.

Using the MIN/MAX methodology, the first two compile passes are performed with medium compile and area effort, while the third pass is a high effort incremental compile, all using a max_area 0 constraint.  This approach is particularly area friendly since the max_area 0 constraint is applied in a divide-and-conquer fashion where it can have more impact, and the high effort is applied in an incremental fashion when timing and constraints have come close to converging.  The core level optimization is performed with a high effort incremental compile once again, but with medium area effort since the area has already been pushed in the bottom up compiles.  The area constraint, equal to the third pass core area results, allows design rules and hold times to be fixed essentially for free with respect to area.  Finally, the full chip is optimized using compile –top without area constraints to fix remaining top level issues without impacting runtime.

- Nitty Gritty Dirt Details

Once a suitable methodology and standardized set of scripts are in place, the fine tuning details can be built in.  The following are some of the nitty gritty dirt details and recommendations for achieving optimal area and timing results.

  - hlo_resource_implementation = constraint_driven

The default value for this variable was changed to use_fastest in the 1999.05 version of Design Compiler.  This new default value will force the use of more area hungry designware implementations, whether they are needed or not.  In a single pass methodology, this may make timing closure easier to achieve, but at the expense of increased area.  The original default, constraint_driven, allows synthesis to choose the smallest implementation that still meets timing.  This approach is much more area friendly, but works best in multi-pass flow.

  - hdlin_use_cin = true

Setting this hidden variable allows a+b+1 to be implemented with the carry in of an adder instead of a separate adder and incrementer, yielding a considerable area benefit.

  - hdlin_don’t_infer_mux_for_resource_sharing = false

This setting works with hdlin_infer_mux = default to insure that a MUX will always be inferred when the infer_mux directive is used, provided that the hdlin_mux_size_limit variable is set high enough for the design.  This would prevent resource sharing when synthetic operators drive more than one branch of a case statement, but if the infer_mux directive is used, this is assumed to be the intent.

  - hdlin_mux_size_limit = 256

This variable works together with the infer_mux directive in the RTL to produce MUX OPs in synthesis.  Depending on the size of the MUXes inferred in your design, the default value of 32 may not be enough.  If this happens, MUXes will still not be synthesized.  Choose a binary value appropriate for your design.

  - auto_wire_load_selection = false

Although automatic wireload selection is convenient, problems can occur during the compile when the gate count exceeds the selected wireload, and the next higher wireload is selected.  Sudden jumps in negative slack occur at this point, and area is consumed in the attempt to overcome the increased delays.  Instead, specify wireloads (custom generated wireloads when possible) hierarchically by design using enclosed mode.  Enclosed mode hierarchical wireload models more closely match the results of timing driven placement without overbuilding the design and consuming excess area.

  - Critical Range and Weighting

Without the use of a critical range, synthesis reverts to a default critical range of zero, which causes synthesis to play “whack a mole” with the most critical path, one path at a time.  If a bogus constraint causes an unfixable timing violation, synthesis will give up, not even looking at the lesser violating paths, even if they could be easily fixed.  This causes timing closure problems that lead to area consumption.  By using a critical range, more violating paths are looked at together, effectively widening the “whack a mole” bat, and reducing any timing closure problems.  If the worst negative slack is less than the critical range after a compile, all the paths have been optimized.

When there are multiple clocks and different frequencies, depending on the design, the fastest clock becomes the speed limit for the design.  Synthesis automatically creates path groups by clock domain, but gives them equal weight, which causes synthesis to treat the magnitude of worst negative slack equally in all clock domains.  If all the clock domains are asynchronous or independent of each other, this is the desired behavior.  If there are divided clocks, a divide by two for example, and both original and divided clocks are used, the slower clock can actually tolerate twice the negative slack as the faster one before it becomes the speed limit for the design.  By creating the paths groups manually after the clocks are declared using the weight option as follows:

group_path –name [clock name] –critical_range [value] –weight [value, default is 1],

the negative slack allocation can be scaled by clock domain, guiding synthesis to expend weighted amounts of effort in each clock domain.  If the faster clock target frequency must be adjusted to meet time to market (assuming a large number of violating paths, and that it even can be), the negative slack scaling can serve to prevent unnecessary area consumption.

  - Ideal Nets and Scan Chain Reordering

In addition to the clock nets, the reset and scan enable nets should also be treated as ideal nets in synthesis.  For the same reasons that clock tree synthesis (CTS) is saved for layout, resets and scan enables should be saved for layout as well.  This greatly reduces routing congestion, which helps improve timing, and allows more aggressive die sizing to take advantage of reduced area in synthesis.  Scan chain reordering should also be done in layout to further reduce routing congestion, as well as greatly reduce design rule buffering of these nets.  Reducing the wire length and capacitance on the scan paths will also help timing on the functional paths (since each flop drives both a functional and a scan path) as well as reduce power consumption for the design.

  - Processing Characterize Write Script Files

When characterize is performed on designs containing other dont_touch’ed designs, the resulting write_script constraint files will set the dont_touch on the instances instead of the designs themselves.  In order to perform core level scan insertion and optimization as well as top level optimization, the dont_touch attributes must be removed.  It is far more straight forward to remove the don’t_touch from designs than from their instances by using the following construct:

foreach (des, find(design)) {

current_design des

remove_attribute current_design dont_touch

}

current_design [original design]

To correct the write script problem, some simple post processing is required.  A simple

grep –v ‘set_dont_touch ’ filename > newfilename

mv newfilename filename

will get the job done.  The space before the ending quote is intentional to keep any set_dont_touch_network commands intact.  Any intentional set_dont_touch’es on cells can be restored by including them in a revised script file that will be included after the write script file on the next compile.

  - Core Level Scan Stitching and Optimization

After the multi-pass bottom up compiles are complete, the core level scan stitching and optimization can begin.  By using the pre-optimization core area as the area constraint, scan stitching, design rule and timing optimization, and hold fixing (if needed), can be done practically for free with respect to area.  Stitch the scan chains by running insert_scan with no design rule or timing optimization, followed by an incremental compile with high map effort and medium area effort (to save run time since high area effort has already been applied to the lower level blocks).  To gain a slight additional area improvement (depending on the design), run reduce_constants between the scan stitching and incremental compile steps.  In addition, set the following variable for core level optimization only:

compile_sequential_area_recovery = true

Finally, DC Ultra optimization capabilities, if available, can be used during this incremental compile for further timing and area improvement, but would have greater area impact in the final high effort incremental compiles of the lower level modules.

  - Prelayout Netlist Creation and Optimization

As previously discussed in the MIN/MAX methodology, the clocks & pads blocks are compiled first, followed by full chip netlist assembly.  Any logic present in the pads block or the top level is handled with grouping as discussed in the selective ungrouping section.  Once the netlist is assembled, the top level scan stitching is performed.  The stitching can be performed manually with scripted create_net and connect_net commands or with insert_scan set up to route serial only without any design rule or timing optimization.  The final prelayout netlist optimization is performed using compile –top with compile_top_all_paths = true.  To fix hold violations, perform this under MIN/MAX conditions with set_fix_hold all_clocks() and add the –prioritize_min_paths switch to the compile (it still works as of the 1999.10-6 version of Design Compiler, if and when they break this, it will probably take a DC Ultra license to set the cost priority to insure holds are fixed).

  - Prelayout Hold Time Fixing

The idea of prelayout hold time fixing mentioned throughout this paper has been a hotly debated topic in the past and requires some very careful consideration before implementing.  The intent is to address known hold time scenarios up front to avoid or reduce any post layout hold surprises that could disturb the layout enough to effect timing closure, time to market and area.  Without very careful analysis and preparation, prelayout hold time fixing could have disastrous results, inserting many tens of thousands of buffers.  It is critical to have accurate predictions of clock tree insertion delay and skew, as well as accurate constraints on the design.

There should be no hold fixing on ideal nets like reset and scan enable, as well as static and asynchronous inputs to the chip.  This can be prevented by unconstraining the min paths as follows:

Set_input_delay –max [value] –clock [clock_name] [input_name]

Remove_input_delay –min [input_name]

For core level dedicated scan chain input ports, use the following after routing scan chains with insert_scan but before optimizing and fixing holds:

Set_false_path –hold –from [scan input ports, usually test_si*]

There should not be any hold time fixing between asynchronous clock domains.  Unconstrain these min paths with:

Set_false_path –hold –from asynchronous_clock1 –to asynchronous_clock2

Within a single clock domain, the acceptable skew must be tight enough to minimize hold problems within that domain, especially at the scan inputs of flops.  Libraries with negative hold time have a great advantage here, allowing higher skew values.

The remaining possibilities include chip inputs to capture registers, registers to chip outputs if applicable, and paths between synchronous clock domains with different insertion delays.  Insertion delay differences arise from separate trees for divided clocks, gated and non-gated clocks, and intentional early and late clocks.  These are the areas where there will almost definitely be hold problems, even after layout.  By addressing these up front, any post layout hold fixing can be greatly reduced.  With timing driven placement, the prelayout hold fixes may very well be optimized out along the way, but the gates required for the real fixes have already been accounted for and last minute surprises are avoided.

- Conclusions and Future Work

By combining a number of factors into an area friendly methodology, timing closure and area reduction can become concurrent goals.  It is important to remember, however, that area is still part of a three-way tradeoff with timing closure and time to market.  The methods and details presented here attempt to strike a balance, and under some circumstances may still leave room for further area improvement.  When presented with the choice between good, fast, and cheap, we should be able to pick somewhere close to three with these methods, and enjoy the benefits of reduced area.

Future area improvement possibilities include the usage of MC in DC (Module Compiler in Design Compiler) by combining Designware Foundation, DC Ultra and Design Compiler 2005 to use partition_dp instead of transform_csa.  Another possibility is the use of Power Compiler RTL clock gating, where MUXes in front of banks of registers are replaced with a latch based clock gating cell to implement clock enabled registers.  Finally, the use of Physical Compiler to eliminate wireload model inaccuracies could have a positive impact on area.  By using the physopt –area_recovery switch under MIN/MAX conditions while fixing hold times during final optimization, an optimal timing and area balance should be achievable.

- References

1.	Akhil Bagri and Chanda Dani; “Getting the Most from Design Compiler for Area & Delay,” SNUG 2000 San Jose Tutorials.

2.	Tom Wilderotter and Jay Adams; “Synthesis 2000:  Design Compiler,” SNUG 2000 Boston Tutorials.

3.	Wiegand, Robert; “Using MIN/MAX Compile In A Multi-pass Synthesis Flow,” SNUG 1999 San Jose Proceedings.

4.	Mohammad Mohsin; “Hierarchical Wire Load Estimation,” SNUG 1997 Proceedings.

5.	Design Compiler Reference Manual, Synopsys Online Documentation Version 1999.10.

6.	Design Compiler Reference Manual, Synopsys Online Documentation Version 2000.05.

7.	Jon Baldry; “A Compile Strategy Using Design Budgeting,” Design Budgeting Applications Note, Synopsys, Inc., October, 1998.