Using MIN/MAX Compile In A Multi-pass Synthesis Flow

Robert B. Wiegand

Ensoniq Corporation

A Wholly Owned Subsidiary of Creative Technology LTD.

ABSTRACT

A flow is presented which takes advantage of MIN/MAX compile capability in a combined bottom-up/top-down flow.  Necessary preparation steps are discussed, as well as synthesis settings to maximize the effectiveness of the MIN/MAX compile.  Some pitfalls and work-arounds are also discussed.

1.0 Introduction

Among the many Challenges of ASIC synthesis, fixing hold time violations can be one of the most frustrating.  If setup time requirements are not met, the design will work at a slower frequency, but if hold time requirements are not met, the design will not work at all.  Typically, hold times are checked as one of the final steps before layout, or perhaps not even considered until post-layout.  If hold time violations are repaired at MIN conditions, setup times may be broken at MAX conditions.  Trying to fix both can lead to many iterations between MIN and MAX conditions.  The problem is made even more difficult with the presence of multiple clocks and internally generated clocks.

By using the MIN/MAX compile capability introduced in the 1998.02 version of Synopsys Design Compiler, the headaches of fixing hold time violations can be greatly reduced.  There are certain key points in the synthesis flow where MIN/MAX capability brings substantial benefit.  Great care must be used however, to avoid some of the pitfalls including area and runtime increase.  This paper presents a flow to get the most benefit from the MIN/MAX compile, and details some of the preparation steps, compile settings, pitfalls and workarounds.

- Top Down vs. Bottom Up Compile

A recent Synopsys white paper titled Synthesis Methodology for Large Designs suggests designers no longer break up designs into 2-10K chunks, but instead perform a top down hierarchical compile.  Ideally, this would be the perfect place to apply MIN/MAX compile capability.  With full visibility of all paths in the design, Design Compiler can attempt to meet setup and hold time requirements together, without having to iterate between operating conditions.  An additional benefit is having only one set of constraints at the top level, instead of constraints for every level of the hierarchy.  However, the data in that paper suggests that the top down compile works best on designs having one clock and few or no timing exceptions.  These guidelines are not always possible to meet.  Long runtimes can be a problem with this method, especially when area constraints are used. Incremental change is not supported – simple RTL changes require the entire design to be resynthesized.

Another approach is the compile-characterize-write_script-recompile method.  This method is very effective with complex clocking and timing exceptions, but requires good initial compile constraints.  Obtaining these initial constraints can be very tedious, and often approaches full blown time budgeting.  Another well known problem is the ping pong effect of characterize (after characterizing and recompiling, first pass setup violations are fixed, only to reveal new setup violations which met timing in the first pass).  Attempting to apply MIN/MAX compile in this bottom up approach could lead to unnecessary area usage if the input delay constraints are not accurate on the first pass, as well as further compound the ping pong effect.

How can we keep the benefits of each approach while avoiding their pitfalls, and find the most effective use of the MIN/MAX compile capability?  The flow presented here combines a modified compile-characterize-write_script-recompile method with a top down approach at the core and top levels to get the most benefit from MIN/MAX compile.  The ping pong effect of characterize is exploited and used to our advantage in this methodology.  The details of each phase of the process are explained in the sections that follow.

- Overall Flow – The Big Picture

The design is partitioned so that the top level instances a clocks block, a pads block, and a hierarchical core(s).  The core level block contains only hierarchy, with all the logic at the leaf levels.  The lower level modules of the core are compiled bottom up, characterized, recompiled, characterized again, and incrementally compiled.  By compiling in three passes, the ping pong effect of characterize is used to our advantage.  This will be further explained in the next section.  Next, the core is compiled incrementally from the top down, using the MIN/MAX compile to fix hold violations as well as any remaining setup violations.  The clocks block is then compiled, if necessary, using MIN/MAX as well.  The pads block contains instanced pad cells and should not require compiling.  If it does, however, the compile is performed at MAX conditions only.  Finally, the top level is analyzed and compiled in_place at MIN/MAX conditions if necessary, using propagated clocks.  Any post layout optimization is also performed using MIN/MAX compile.

Figure 1.  Overall Flow

- Exploiting the Ping Pong Effect

When a module is characterized, it is within the context of the rest of the design.  Any positive or negative timing slack relating to paths through that module are reflected in the resulting constraints.  Characterize assumes all negative slack through the characterized module must be made up entirely within that module.  Conversely, any positive slack can be distributed entirely within that same module.  If one were attempting to meet timing on a fully compiled hierarchical design by attacking it one module at a time in a hand-to-hand combat fashion, characterize would be the weapon of choice.  This, however is a tedious manual process.

What happens if we characterize modules in parallel?  All positive or negative slack on paths through a particular module will be addressed entirely within that module.  When a path passes through multiple modules, it’s positive or negative slack is addressed multiple times.  As an example, let us consider the two paths in figure 2a.  Path 1 and path 2 each pass through module A and module B with a total timing requirement of 10ns.  Path 1, taking 4ns through each block, has a total positive slack of 2ns.  Path 2, taking 6ns through each block, has a total negative slack of 2ns.  If module A and B are characterized in parallel, the resulting constraints will have both module A and B trying to make up the 2ns negative slack for path 2, while allowing path 1 to be 2ns slower.  If these conditions are met in the second compile, path 2 has positive slack, since it is now 2ns faster through each module.  Path 1, however, was allowed to be 2ns slower in each module and now has 2ns total negative slack.  This is shown in figure 2b.  If we characterize and compile once again we should wind up where we started.  This is the ping pong effect.  If, however, we do an incremental compile instead, we are able to tell the tool “if it ain’t broke, don’t fix it!”  This will restore path 1 to its original condition of 2ns positive slack, while keeping the 2ns positive slack result on path 2 from the second pass.  This is shown in figure 2c.

Figure 2.  Exploiting The Ping Pong Effect

This may sound like a compelling argument for the use of the design budgeting feature in the 1998.08 version of Synopsys, or even full manual time budgeting, but how are we to know which module is able to make up the 2ns of negative slack from the first pass compile?  Design budgeting or manual time budgeting may assume each module can make up 1ns, but what if module A cannot make up any slack while module B can make up 2ns?  By exploiting the ping pong effect of characterize in this three pass flow, we are able to:

- identify the critical and non-critical paths in the first pass

- set aside the non-critical paths in the second pass and attempt to fix the critical paths in each block they pass through

- restore the non-critical paths in the third pass

At the core level, the three pass compiled results should meet timing or come very close.  This is an ideal place to apply the MIN/MAX compile capability.  Since setup timing is met or is very close, the tool does not have to spend a lot of effort attempting to meet setup requirements.  The tool can, however fix the hold times while keeping the setup timing requirements accounted for.

- Module Level Flow

The justification for the three-pass module level flow was explained in the previous sections.  The details of each pass are explained here.  During all three passes, the modules are compiled with a max_area 0 constraint.  At the module level, this allows the quality of result (QOR) benefits without the significant runtime penalty encountered if max_area 0 is attempted in a top down compile.  The compiles are performed with worst case or MAX conditions to concentrate on meeting setup time requirements.  In addition, the inputs of all modules are constrained to a max_fanout of 1 for the first two passes.  This prevents any high fanout surprises at the core level or below.  After the second characterize, the drive cell and output load constraints are appropriately matched for each module, so the max_fanout constraint can be removed from the inputs for the third pass compile.

- First Pass Bottom Up Compile

The first pass bottom up compile is performed using default constraints and a medium effort compile.  The default constraints include worst case operating conditions, appropriate wire load, ideal clock definitions including predicted insertion delay and skew, driving cell and loading on the ports, and input and output delays.  Instead of over-constraining at the module level, the default constraints allow for clock to q timing plus a small wire load delay for the inputs and register setup time plus a small wire load delay for the outputs.  This assumes all outputs are registered and if they are not, they are registered at the input of the next module.  Combinatorial paths through multiple modules will be under-constrained as a result.  The scripting environment used to implement this methodology allows for overriding these default constraints when necessary.  By embedding the exceptions directly in the RTL, the exceptions will apply for the first pass only.  By using “plug-in” exception files, the exceptions can be applied first pass only, or as a global override for all three passes.

Before saving the compiled results of each module, any compile-generated hierarchy (mux-ops, designware, etc.) is ungrouped and a dont_touch attribute is placed on the design.  Only the current design is saved, since any underlying hierarchy should consist only of previously compiled designs with the dont_touch attribute applied.  Complex timing requirements and exceptions such as false and multi-cycle paths are reserved for the core level and characterized down the hierarchy for the second pass compile.  The core RTL file is read in, constraints and timing exceptions are applied, and the core_name.db file is written out along with timing reports.  No compile performed on the core at this time.  This first pass serves as a design exploration or feasibility compile and identifies the critical and non critical paths.  If the setup time is way off after all appropriate timing exceptions have been applied, the source of the problem should be identified and fixed in the RTL.  All endpoints in the design should be constrained from the core level at this point as confirmed by a check_timing command.  If there are any unconstrained endpoints, max_delay constraints should be applied to resolve them.  Once these steps are completed, a top down parallel characterize is performed.

- Second Pass Bottom Up Compile

The second pass bottom up compile is also performed using a medium effort compile starting from the RTL.  The default constraints are applied and then overridden with the write_script constraints from the first pass characterize.  Incremental design changes to the modules can be introduced in this pass since any new input or output ports will be covered by the default constraints (the scripting environment supports incremental design changes).  The critical paths from the first pass will be attacked in this pass, while the non-critical paths will most likely be broken.  Once again, a top down parallel characterize is performed

- Third Pass Bottom Up Compile

The third pass bottom up compile is performed on the second pass db files using a high effort incremental compile at MAX conditions.  Once again, default constraints are applied and then overridden with the write_script constraints from the second pass characterize.  Since all compile generated hierarchy was ungrouped before the db file was saved in the previous compiles, any boundary effects will be optimized out in this step.  The -incremental_mapping switch will insure that the timing improvements gained in the second pass are maintained and only the paths that still violate will be changed.  Any paths that met timing in pass 1 but violated in pass 2 will be restored in pass 3.  Any paths that violated in pass 1 and were improved but still violated in pass 2 will be further addressed in pass 3.  With the high effort compile, these paths will have the additional benefit of critical path resynthesis (with a DC Expert license) and logic duplication (with a DC Ultra license).

- Core Level Flow

With the completion of the three-pass compile flow, setup time requirements should be met or very nearly met at the core level.  The core is now ready for a top down high effort incremental compile with MIN/MAX conditions to fix hold time violations and any remaining setup time violations.  The core_name.db file is read in and linked to pull in the hierarchy.  The dont_touch attribute is removed from all designs, and then a uniquify if performed.   A set_fix_hold is applied to all clocks and the variable timing_self_loops_no_skew is set to true.  A max_area 0 constraint can be applied here to really grind down the area, but the run times can be excessive, especially with very rich libraries.  A more reasonable approach is to apply the area result of the third pass compile as the max_area constraint.  This way, the tool can attempt to fix any remaining setup violations as well as hold violations while minimizing any area or runtime penalty.  When the compile is completed, the results are saved hierarchically in a single db file.

Depending on the design and the library, there may be some hold violations remaining when this step is complete.  In this case, a compile –prioritize_min_paths –only_design_rule is performed on top of the previous result.  Due to some limitations of the MIN/MAX timing capability and the workarounds used, the results of the core level compile should be analyzed at MIN, TYP, and MAX conditions as well as MIN/MAX.  In some cases, it may be necessary to perform the compile –prioritize_min_paths –only_design_rule step at MIN conditions.  The limitations of MIN/MAX and their workarounds will be discussed in detail in a later section.

Figure 3.  Core Level Flow

6.0 Top Level Flow

Once the core level optimization is complete, the top level processing can begin.  Since the top level instances the pads and clocks blocks as well as the core, we will discuss them here as well.

6.1 Pads Block

The pads block is fairly straight forward, since it contains instantiated technology specific pad cells.  Some times it is necessary to invert signals in the pads block when converting from one library to another.  For example, tri-state enable signals may be active high in one library and active low in another.  Rather than change to logic in the core to accommodate this situation, simple inversions can be tackled in the pads block.  Rather than hand instantiate inverters, they may be written into assign statements or (gulp!) inverted on the ports of the cells.  The compile itself largely resembles the first pass compile at the module level with a few variations. First, a dont_touch attribute is placed on all instantiated cells.  Next, the following command is performed to move all unmapped logic to a new level of hierarchy:

group –logic –except find(cell,{list of instantiated cells}) –design pad_logic –cell pad_logic

The new design, called pad_logic, is compiled with default constraints at MAX conditions and then ungrouped back into the pads block.  No compile is actually performed on the pads block itself.

6.2 Clocks Block

The clocks block contains technology specific clock tree macro cells or buffers, as well as macro cells or buffers for the global reset.  Any internally generated or divided clocks are handled here, as well as any required clock gating or muxing.  This way, all the “evil” is confined to one block where it can be managed more easily.  If all cells and logic are instanced, no compile is necessary.  If all logic requiring a compile is purely combinatorial, a compile at MAX conditions is performed.  If any logic is sequential, a compile is performed at MIN/MAX conditions, with set_fix_hold on all relevant clocks.  First, like the pads block, a dont_touch attribute is placed on all instanced cells.  Specific clocks only need to be declared for this block if they drive sequential logic, otherwise all paths through this block are purely combinatorial and can be constrained relative to a virtual clock.  Any specific clocks required should be propagated if there is any clock gating logic involved.

6.3 Top Level Pre-Layout IPO

With the completion of the clocks and pads blocks, as well as the optimized core, we are ready to move on to full top level netlist analysis and optimization.  To create the full gate level netlist, no compile should be necessary since all logic has been mapped and optimized previously.  The optimized core is read in, followed by the top level RTL file.  The pads and clocks blocks are linked in, and then the dont_touch attribute is removed from all designs in the hierarchy.  If flattening is required by the layout tools, that step is performed here.  Next, a change_names is performed to comply with layout tool naming rule requirements.  At this point, the full chip gate level netlist is written out.

The constraints from the core level are translated and applied to the top level, but this time with propagated clocks.  Internally generated clocks must have MIN and MAX insertion delays applied to the clock source point as well.  Any additional top level constraints are also applied.  Timing is analyzed at MIN, TYP, MAX and MIN/MAX conditions.  If the input, output, and clock insertion delay values were reasonably accurate at the core level, there should only be minor violations here.  Otherwise, these values should be updated so the core can be reoptimized.  Expected violations at this point include fanout related violations, as well as minor setup and hold violations seen now because of the propagated clocks.  Minor violations can be repaired with an in-place optimization (IPO) at MIN/MAX conditions with no area constraints.  Leaving out area constraints keeps the changes minimal and the run times reasonable.  A set_fix_hold is applied to all clocks, and the variable timing_self_loops_no_skew is once again set to true.  Since this is a pre-layout netlist, we can allow the tool more degrees of freedom than we would tolerate with a post-layout netlist.  These include Boolean cell swapping as well as cell resizing, and buffer inserting and deleting.  The compile command and related variable settings are as follows:

compile_update_annotated_delays_during_inplace_opt 	= true

compile_dont_touch_annotated_cell_during_inplace_opt	= false

compile_ignore_footprint_during_inplace_opt 		= true

compile_ok_to_buffer_during_inplace_opt 			= true

compile_ignore_area_during_inplace_opt			= true

compile_disable_area_opt_during_inplace_opt		= true

compile –in_place –prioritize_min_paths (-only_design_rule is optional)

The results are once again analyzed at MIN, TYP, MAX, and MIN/MAX conditions.  It may be necessary to run the IPO again at MIN conditions with the –only_design_rule switch to fix any remaining hold violations.

Figure 4.  Top Level Pre-Layout IPO

6.4 Top Level Post-Layout IPO/LBO

Once layout is complete, the results need to be analyzed again.  In addition to a gate level netlist with clock trees, there should be an SDF file to specify post-layout delays, a set_load script to specify post-layout net capacitance, and a PDEF file to specify post-layout cell locations and obstructions.  Once again, top level constraints are applied with propagated clocks.  Insertion delays for internally generated clocks will have to be verified with static timing analysis at MIN, TYP and MAX conditions.  Next, the set_load script is included to back annotate the capacitance, followed by a read_timing command with the appropriate min_triplet and max_triplet switches to back annotate timing for the desired operating condition.  Timing is analyzed at MIN, TYP, MAX and MIN/MAX.  What happens next depends on the degree of violations, availability and quality of post-layout data, and the tools available.

6.4.1 Floorplan Manager Flow

If Floorplan Manager is available, and the SDF and PDEF files and set_load scripts are complete, several options are available.  If the timing is reasonably close, Location Based Optimization (LBO) can be used to make the necessary changes.  The design is read in, constraints and propagated clocks are applied, post layout data is back annotated for MIN/MAX conditions, and wireload models are generated and applied to the design. The compile command and related variable settings are as follows:

compile_update_annotated_delays_during_inplace_opt 	= true

compile_dont_touch_annotated_cell_during_inplace_opt	= false

compile_ignore_footprint_during_inplace_opt 		= false

compile_ok_to_buffer_during_inplace_opt 			= true

compile_ignore_area_during_inplace_opt			= true

compile_disable_area_opt_during_inplace_opt		= true

reoptimize_design_disable_area_during_postlayout_opto	= true

reoptimize_design_changed_list_file_name			= [desired file name]

lbo_buffer_insertion_enabled					= true

lbo_buffer_removal_enabled					= true

lbo_cells_in_regions						= false

lbo_lfo_enable_at_pin_count					= 3	/* 3 is default */

reoptimize_design –in_place –prioritize_min_paths (-only_design_rule is optional)

The above settings attempt to fix the design with the least amount of disturbance, using placement information to insert or remove buffers as necessary.  The -in_place switch limits the LBO to cell resizing and buffer insertion and removal.   The –prioritize_min_paths switch elevates hold violations to design rule status giving them a higher priority than setup violations.  To accompany the change list written out automatically, a write_clusters –new_cells_only is performed to create an incremental PDEF file.  An SDF file is written out as well, since the starting SDF data will not reflect the changes.  Once again, the results are analyzed at MIN, TYP, MAX, and MIN/MAX conditions.  If there are hold violations at MIN, the above process can be repeated at MIN conditions using the generated SDF file and adding the –only_design_rule switch.

If there are still violations at MIN/MAX, the next level of change is to replace the –in_place switch with –post_layout_opto.  This enables location based fanout optimization and causes a higher degree of change to the netlist.  If this is still not enough to fix all the problems, the –map_effort high switch can be added to trigger critical path resynthesis.  This will cause the highest degree of change to the netlist.  Finally, if the timing is way off, the best choice may be to start over from the RTL with the newly created wireload models.

If the LBO-1 message does not appear when mapping optimizations start, there is a problem with the back annotated data and LBO will not be enabled.  The check_design –post_layout command can be used to identify the problem.  If the problem cannot be resolved, a non-LBO method must be used.  Without LBO capability, the procedure is the same as above, but any delay calculations on changed nets and cells will be based purely on the newly created wireload model instead of the back annotated data.  Since placement data is not used, no incremental PDEF file is needed.

Figure 5.  Post-Layout Floorplan Manager Flow

6.4.2 Non-Floorplan Manager Flow

If Floorplan Manager is not available, the post-layout optimization procedure is considerably different.  The SDF file and set_load scripts are still used, but the PDEF file is not.  Top level constraints, propagated clocks and internal clock insertion delays are handled as before, and timing is once again analyzed at MIN, TYP, MAX and MIN/MAX conditions.  Since any netlist changes will replace annotated timings with wireload model timings, special care is required to fix hold times.  The optimization is performed in two passes.  The first pass fixes hold times at design rule status using only buffer insertions.  All annotated delays are locked in place to avoid wireload model inaccuracies causing the tool to miss any hold violations. The compile command and related variable settings are as follows:

compile_update_annotated_delays_during_inplace_opt 	= false

compile_dont_touch_annotated_cell_during_inplace_opt	= true

compile_ignore_footprint_during_inplace_opt 		= false

compile_ok_to_buffer_during_inplace_opt 			= true

compile_ignore_area_during_inplace_opt			= true

compile_disable_area_opt_during_inplace_opt		= true

reoptimize_design_changed_list_file_name			= [desired file name]

compile –in_place –prioritize_min_paths -only_design_rule

The second pass attempts to fix setup times without breaking any hold times using only cell resizing. The compile command and related variable changes are as follows:

compile_update_annotated_delays_during_inplace_opt 	= true

compile_dont_touch_annotated_cell_during_inplace_opt	= false

compile_ok_to_buffer_during_inplace_opt 			= false

compile –in_place –prioritize_min_paths

Figure 6.  Post-Layout Non-Floorplan Manager Flow

The change list file generated contains two sections.  The first section contains buffer insertions for hold time fixes, while the second section contains cell swaps for setup time fixes. The resulting netlist is written as well as an SDF file, and timing is analyzed at all conditions.  If there are hold violations at MIN, the first pass can be repeated at MIN conditions using the generated SDF file.

7.0 Key Points

There are a number of key points and issues that contribute to the successful use of this methodology and the MIN/MAX compile capability.

7.1 Feasibility Compiles

Engineers designing at the block level MUST run synthesis as part of the validation of the block.  A simple first pass compile with default constraints at MAX conditions is all it takes to find and fix any major surprises up front.  Any special timing constraints or exceptions should be imbedded in the source code.  The RTL should not be checked in to source control until it passes simulation AND synthesis – without any special heroics (we’ll save the heroics for full chip integration).

7.2 All Endpoints Must Be Constrained

The check_timing command should be used to identify any unconstrained endpoints in the design so they may be resolved.  Unconstrained endpoints may result from false_path commands being too general, sequential library elements (synchronous RAMs for example) having incomplete timing arcs, or clocking mistakes in the RTL.

7.3 Clock Considerations

In order to use the MIN/MAX compile capability successfully, it is crucial to use the most accurate clock information possible.

7.3.1 Accounting For Insertion Delay and Skew

As discussed in previous sections, any clock related logic should be placed in the clocks block which is instanced in the top level.  This logic may include gating, muxing, and clock tree placeholder buffering.  This allows the use of ideal clocks in the core.  Insertion delay and skew must be accounted for in order to use MIN/MAX to accurately fix hold times before layout, especially with multiple clock domains and crossings between them.  Fixing hold time violations before layout greatly reduces the magnitude and number of post-layout iterations.  For ideal clocks, use the following set_clock_skew switches:

-plus_uncertainty			clock skew relating to hold times

-minus_uncertainty			clock skew relating to setup times

-min –delay				minimum total clock insertion delay

-max –delay				maximum total clock insertion delay

The insertion delay is the total delay from the ASIC pad through any clock muxing, gating, and buffering logic to the clock port of the core.  The delay through the cells used as clock tree placeholders must reasonably match the expected delay through the final clock tree.  The additional delays of the pads and gating logic must be determined through static timing at MIN, TYP, and MAX conditions.  The first pass compile results together with the clocks, pads and top level blocks are used to obtain these timing numbers (reasonable guesses are used at first).  The total delay from pad to output of clock tree placeholder under MIN, TYP, and MAX conditions are used for both the –min –delay and –max –delay switches under MIN, TYP, and MAX conditions respectively.  For MIN/MAX conditions, use the MIN total delay for –min –delay , and the MAX total delay for the –max –delay.  For top level analysis, the clocks are propagated, so clock tree delay is accounted for, as long as the place holder buffers have reasonably accurate delays.

The allowable clock skew must be specified up front, and must be met by the clock tree insertion tools during layout.  Additional hold time margin can be added if required by increasing the plus_uncertainty.  For top level pre-layout analysis, the uncertainties still apply, but they should be set to 0 for post-layout analysis.  Finally, don’t forget the set_drive 0 on the clock port, set_resistance 0 on the clock net and set_dont_touch_network on the clock.

7.3.2 Internally Generated Clocks

Since Design Compiler cannot calculate the period and insertion delays of internally generated or divided clocks, they must be specified manually.  Let’s take a simple example of a divide by two clock.  The logic in the clocks block would consist of a toggle flip flop and a clock tree place holder buffer.  The insertion delay to the clock pin on that flop is handled as discussed above.  There is additional delay from the clock-to-q of the flop plus the delay through the buffer.  These numbers must also be obtained through static timing analysis at MIN, TYP, and MAX as discussed above.  The total insertion delay at the core level is equal to the sum of the delay from the external clock pin to the clock pin of the dividing register, the clock-to-q delay of the register, and the delay from the output of the register to the output of the buffer.  These delays are applied to the ideal core level clocks with the –min –delay and –max –delay switches, along with the uncertainty values as described in the previous section.

For top level analysis using propagated clocks, once again Design Compiler cannot calculate the result of the clock dividing logic.  Since propagating the clock will pick up the buffer or clock tree timing, the clock is declared on the output of the dividing register as follows:

create_clock –name [divided clock name] find(pin,[dividing register cellname/Q])

-period [divided clock period] –waveform {[divided clock rise], [divided clock fall]}

By declaring a clock on an output pin of a register, the path is segmented and starts at time zero.  The insertion delay to this new clock point is the total delay from the outside clock pin to the dividing register clock pin plus the clock-to-q delay of the register.  The set_input_delay command is used to handle this insertion delay as follows:

set_input_delay –min [minimum insertion delay value] [dividing register cellname/Q]

set_input_delay –max [maximum insertion delay value] [dividing register cellname/Q]

The values are obtained with static timing analysis at MIN, TYP, and MAX conditions.  The resulting delay values are used for both the –min and –max switches under MIN, TYP, and MAX conditions respectively.  For MIN/MAX conditions, the MIN delay value is used with the –min switch and the MAX delay value is used with the –max switch.  The plus and minus uncertainty values are also applied for pre-layout analysis, but set to zero for post-layout analysis.

7.3.3 Asynchronous Clock Domains

If there are non-harmonically related clock domains in the design, special care must me used in the interfaces between them.  Assuming metastability is accounted for, the other issue is that Synopsys attempts to find a common period between the two, resulting in huge clock periods.  One practice is to just make the periods harmonically related, and alter the plus and minus uncertainty values to compensate.  Another method is made possible in Synopsys version 1998.02.  Beginning with this version, if the two clock domains have set_false_path between them, the destination register inputs are revealed as unconstrained endpoints with the check_timing command.  Using static timing, a reasonable value can be found to assign as a max_delay from the source register in clock domain 1 to the destination register in clock domain 2.  By setting the max_delay, the endpoint is now constrained as shown by the check_timing command.  The commands are as follows:

set_false_path –from clock1 –to clock2	/* reg2/D is now unconstrained */

set_max_delay [delay value] –from reg1 –to reg2

/* reg2/D is now constrained */

Since the reg2/D endpoint is now constrained only by the max_delay constraint, the constraints will characterize cleanly in the three-pass flow.

It is important to note that in version 1998.02, cell to cell max_delay constraints will override clock to clock false path constraints.  In version 1998.08 this behavior was changed (without any change in the documentation or mention in the release notes).  With the above method, the unconstrained endpoints cannot be resolved.  A workaround is to use a max_delay constraint between clock domains instead of false_path.  The value used is equal to the delay value used previously.  This presents several new problems.  First, if there were non-mission mode paths (i.e. test mode only paths) between the two domains with timing longer than the max_delay value used, these will now become setup violations.  Depending on the nature of the design, there may be many more of these than there were unconstrained endpoints previously.  They must be overridden with a max_delay constraint value equal to the source clock period from the source clock to the destination register as follows:

set_max_delay [delay value] –from find(clock,{clock1}) –to find(clock,{clock2})

set_max_delay [clock1 period] –from find(clock,{clock1}) 	–to [violating cell]

The second problem is the existence of paths constrained by both normal single cycle timing and by max_delay constraints.  These paths tend not to characterize cleanly, causing the need to revise the write_script constraints from characterize.  A workaround for this situation is to use the original false path constraints at the core level for the first two passes of the bottom up compile so that the characterize results will be clean. When the third pass compiles are complete, switch to the new max_delay constraints at the core level.  There will be unconstrained endpoints during the module compiles, but the new constraints will be used for the core level and top level MIN/MAX compiles, fixing any problems the unconstrained endpoints may have caused.

7.4 MIN/MAX Over-pessimism

One major problem with the MIN/MAX capability is that setup and hold calculations are overly pessimistic as documented in the SOLV-IT article Synthesis-304.html.  This article describes the problem with propagated clocks, but the problem exists with ideal clocks as well.  The default behavior places best case and worst case clock tree propagation in the same path, causing excessive setup and hold violations.  The behavior is described in the table below:

| Timing Calculation | Data Path Description | Source Register Clock Delay | Destination Register Clock Delay |
| --- | --- | --- | --- |
| Ideal Setup | worst case longest | -max -delay value | -min -delay value – minus_uncertainty |
| Ideal Hold | best case shortest | -min -delay value | -max -delay value + plus_uncertainty |
| Propagated Setup | worst case longest | worst case MAX | best case MIN |
| Propagated Hold | best case shortest | best case MIN | worst case MAX |

Table 1.  Default MIN/MAX behavior

The workaround is to use the hidden variable remove_min_max_pessimism = true.  The behavior is changed as described in the following table:

| Timing Calculation | Data Path Description | Source Register Clock Delay | Destination Register Clock Delay |
| --- | --- | --- | --- |
| Ideal Setup | worst case longest | -max -delay value | -max -delay value – minus_uncertainty |
| Ideal Hold | best case shortest | -min -delay value | -min -delay value + plus_uncertainty |
| Propagated Setup | worst case longest | worst case MAX | worst case MAX |
| Propagated Hold | best case shortest | best case MIN | best case MIN |

Table 2.  MIN/MAX behavior with hidden variable workaround

For ideal clocks, this is the correct behavior.  For propagated clocks, what we really want is as follows:

| Timing Calculation | Data Path Description | Source Register Clock Delay | Destination Register Clock Delay |
| --- | --- | --- | --- |
| Propagated Setup | worst case longest | worst case MAX | worst case MIN |
| Propagated Hold | best case shortest | best case MIN | best case MAX |

Table 3.  Desired MIN/MAX behavior for propagated clocks

The solution we have can be overly optimistic for some designs.  To work around this, be sure to run all setup and hold checks at MIN, TYP and MAX conditions as well as MINMAX after running any core or top level optimization.  Use the following min_triplet and max_triplet combinations when annotating from the SDF file with the read_timing command:

| Operating Conditions | -min_triplet | -max_triplet |
| --- | --- | --- |
| MIN | minimum | none |
| TYP | typical | typical |
| MAX | none | maximum |
| MIN/MAX | minimum | maximum |

Table 4.  Switch combinations for read_timing command

It may be necessary to follow the MIN/MAX optimization with a –only_design_rule -prioritize_min_paths optimization at another operating condition to completely fix the design.

7.5 Scan Implications

There are a few things to be considered when adding full scan to this methodology.  First, the feasibility compiles should include scan rule checking, and a compile –scan should be performed successfully before the design is checked in to source control.  For the three-pass flow, the –scan switch can wait for the third pass to conserve license usage.  The scan chains should be routed at the core level before the MIN/MAX optimization step to get rid of the self looping paths on all the flip flops.  The test_se (scan enable) and test_mode signals should be routed through the clocks block so they can be buffered for fanout control.  Any rerouting of clocks for test purposes is performed in the clocks block.  If there is any sequential logic in the clocks block, the –scan switch should be added to the compile, and an insert_scan should also be performed.  The pads block should be compiled with the –scan switch followed by an insert_scan to generate any required bidirectional control logic.  Any final global scan signal routing is performed at the top level.

7.6 Floorplan Manager – It’s Not Just For Breakfast Anymore!

The previous discussion of floorplan manager was for the final stages before tapeout.  For large designs in deep-submicron it was assumed that the floorplanning/wireload creation iterations had already occurred. Smaller designs and larger technologies can greatly benefit from the use of Floorplan Manager as well.  Even if no floorplanning is needed and vendor wireload models are satisfactory, the IPO/LBO capabilities combined with MIN/MAX compile capability can greatly reduce the final iterations before tapeout.

8.0 Conclusion

Using the MIN/MAX compile capability of Synopsys can greatly ease the task of fixing hold violations before and after layout.  While MIN/MAX is most effective in a top down approach, a modified bottom up approach is most effective in quickly preparing the design for the MIN/MAX compile while supporting incremental design changes.  Using MIN/MAX with Floorplan Manager after fixing hold times pre-layout is very effective in reducing the number and magnitude of post-layout iterations.  By combining the best points of several methodologies, and using some of the tools and features in unconventional ways, a flow has been developed to more easily converge on setup and hold requirements.

9.0 References

- Chan, Don and Runowicz-Smith, Susan; “Synthesis Methodology for Large Designs, Design Compiler 1997.01 Release,” Synopsys, Inc., June 1997

- Chung, Robert; “Test Plan for M-TIM-MIN Simultaneous MIN/MAX Timing,” Synopsys, Inc., September 1997

- “How to Define Internal Clocks for Synthesis Then for Timing Analysis,” Synopsys, Inc., July, 1997, SOLV-IT! SYNTH-482094.html

- “Back-Annotated Clock Skew is Too Pessimistic Using Minimum and Maximum Compiles,” Synopsys, Inc., August, 1998, SOLV-IT! Synthesis-304.html