High Performance Synthesis using Design Compiler

Release 1.0

Sept. 1999

Author:

Jeff Johnson – Sr. Applications Consultant

SYNOPSYStm

Table of Contents

1	Abstract	1

2	Introduction	1

3	RTL Coding for Performance	1

3.1	Place the critical path in a single level of hierarchy	1

3.2	Analyze the levels of logic before synthesis	1

3.3	Structure logic for late arriving signals	2

3.4	Register outputs of compile blocks	3

3.5	Instantiate complex DesignWare parts	3

4	Developing a Compile Strategy	3

4.1	Top-down vs. bottom-up compiles	3

4.1.1	Compiling top-down	4

4.1.2	Compiling bottom-up	4

4.1.2.1	Creating design-budgets for bottom-up compiles	4

4.1.2.2	Design-budgeting vs. characterize	5

4.1.2.3	The Do’s and Don’ts of dont_touch’s	5

4.2	Baseline compile approach	6

4.3	Using appropriate wire-load models	6

4.4	Using a critical range	6

4.5	Impact of set_max_area on timing	7

4.6	Using boundary optimization	7

4.6.1	Issues with boundary optimization	8

4.7	Using SOP Flattening	8

4.8	Use the fast DesignWare parts	9

4.9	Fast Top-Level Incremental Compiles	10

4.9.1	Recommended flow to reduce run-time of incremental compiles	10

4.9.2	Dont_Touch’s on Sub-blocks	11

4.9.3	Variables for pre-1998.02 Design Compiler	11

4.10	DC Ultra	12

4.11	Using BOA and BRT	12

5	Post-compile Analysis and Re-optimization	13

5.1	How bad is it? – Gathering metrics	13

5.2	Bottleneck analysis	15

5.3	Re-optimization of sub-blocks	15

5.3.1	Optimization with updated design budgets	16

5.3.2	Optimization with characterized constraints	16

5.3.2.1	Characterizing sub-blocks	17

5.3.2.2	Characterizing DesignWare parts	17

5.4	Ungroup hierarchy	18

5.5	Improving high fanout nodes	18

5.6	Implicit dont_touch’s	18

5.7	Look at the DesignWare implementations selected	19

5.8	Direct where Design Compiler focuses	19

5.9	Update the sub-block drive and loading constraints	20

6	Example Scripts	21

6.1	Synthesis experimentation script	21

# Abstract

This paper identifies strategies for producing high performance designs in an RTL synthesis flow using Design Compiler. Three major themes are covered:

- RTL coding for performance

- Developing a compile strategy

- Post-compile analysis and re-synthesis.

Topics are covered in detail with theoretical discussion of why the techniques work and practical examples of how to use them.

# Introduction

Synthesis is a complex process.  Much folk-lore has built up over time regarding how to synthesize high performance designs.  This paper attempts to bring forward the techniques which have proven successful and are backed by a theoretical understanding of how Design Compiler works.

Meeting timing of a high performance design does not occur by accident.  A methodical approach must be taken which attempts to optimize each step in the flow.  Early in the project steps must be taken to ensure the RTL code is good and the design well partitioned.  Accurate constraints must be developed, and a good synthesis strategy followed.  After synthesis a designer must understand how to interpret reports and make necessary corrections to meet timing.

This paper focuses strongly on the 1998.02 and 1998.08 releases.  The 1998.02 release of Design Compiler is, on average, 3-5x faster then previous releases.  Also circuit performance is, on average, 13% better.  The 1998.08 release introduces Design Budgeting.  These and other advantages make a strong case for projects to move to the latest release.

The author would like to thank the following people for their valuable feedback.  Jeff Flieder, Brad McClendon, Terry O’Brien, Erik Olson, Tony Riccobono, Jon Sarri, George Taglieri, and Jeff Winkler.  Any feedback on this application note can be sent to jeffj@synopsys.com.

# RTL Coding for Performance

This section provides an introduction to RTL coding and partitioning for high performance.  For more details, refer to “HDL Coding Styles: Synthesis Application Note” on the on-line docs, and SNUG’98 tutorial “High Performance RTL” at http://www.synopsys.com/news/pubs/snug/snug98_tutorials.

## Place the critical path in a single level of hierarchy

Ensure that the critical path passes through the fewest levels of hierarchy as possible.  Hierarchy is a barrier to optimization.  If the critical path is contained within a single level of hierarchy, Design Compiler has full freedom to optimize it.  Each level of hierarchy that a path crosses limits the optimization flexibility; logic can not be shared between hierarchy.

This principle is related to block size.   If the RTL is divided into many small blocks, paths will naturally cross many levels of hierarchy.  Therefore strive to avoid unnecessarily small blocks.  If they must exist, use Design Compiler’s ‘ungroup’ command to dissolve the small hierarchy before synthesis.  The resulting netlist will have fewer levels of logic.

## Analyze the levels of logic before synthesis

RTL Analyzer can examine the quality of RTL code before synthesis.  For each path, RTL Analyzer will show the implied levels of logic, and the levels of logic per nano-second of available time.  This can be used as a gating factor, determining when a block is ready for synthesis.  A project wide goal can be set for the maximum acceptable levels of logic as reported by RTL Analyzer.  A designer can determine this by synthesizing several blocks and comparing the RTL Analyzer reported implied levels of logic per constraint to the resulting critical path delay.  This comparison can be made within RTL Analyzer.  By averaging a number of paths, a goal can be set for a particular technology library (e.g. “4 levels of logic per nano-second of constraint in RTL Analyzer will usually allow us to meet timing”).  Analysis of the source code can be performed much quicker than synthesis.  Therefore, coding problems can be found and corrected quickly using this RTL analysis methodology.

## Structure logic for late arriving signals

An input to an IF or CASE statement that arrives later than others may cause a timing violation if not properly coded.  These late arriving inputs must be identified by the user.  Several methods can be used reduce the amount of logic that late arriving inputs propagate through.

CASE and IF statements involve two types of signals: select signals and data signals.  When a CASE statement is used, delay is the similar for select and data signals.  When a single IF statement is used, priority encoding is performed on the select signals, but each of the data signals has a similar delay.  A single multi-branched IF statement can be broken into multiple single-branch IF statements.  This results in priority encoding of both the select signals and the data signals.  Therefore, by using a CASE, single IF or multiple IF structure, the designer can determine if the select and data inputs are priority encoded.  Priority encoding should be used for signals that arrive late to the control logic, with the late arriving signal occupying the first branch for single IF’s and in the last IF for multiple IF statements.  Table 1 summarizes the encoding of control structures.

Table 1: Summary of control logic encoding

| Structure | Data input encoding | Select input encoding |
| --- | --- | --- |
| CASE | No encoding | No encoding |
| Single IF | No encoding | Priority encoding |
| Multiple IF’s | Priority encoding | Priority encoding |

Decode logic is easier to code as a CASE statement than a long IF statement.  However, a late arriving data signal can still be accommodated when using a CASE.  Remove the late arriving data signal from the CASE statement.  Next add an IF statement below the CASE, including the data signal which was removed.  The data signal will be muxed into the path at the front of the logic cone.  Many other examples can be found in “HDL Coding Styles: Synthesis Application Note”.

Table 2: Mixing priority encoding with a CASE statement

| Similar delay for all data signals | How to speed up signal s2 |
| --- | --- |
| CASE state IS WHEN s1 => Z <= a; WHEN s2 => Z <= b; WHEN s3 => Z <= c; WHEN OTHERS => Z <= d; END CASE; | CASE state IS WHEN s1 => tmp <= a; WHEN s3 => tmp <= c; WHEN OTHERS => tmp <= d; END CASE; IF (state = s2) THEN Z <= b; ELSE Z <= tmp; END IF; |

Develop accurate input delay constraints for late arriving signals.  While it is easy to give all inputs a uniform arrival time, this will result in sub-optimal synthesis results.  With accurate constraints, Design Compiler can slow early arriving signals to the benefit of late arriving ones.

## Register outputs of compile blocks

Paths that cross hierarchy present problems for constraint generation and synthesis quality.  There are two fundamental issues at work.  First, manually creating I/O delay constraints is guesswork.  The solution to this issue is automatic Design Budgeting, which is discussed in Section 4.1.2.1.  Second, even with good constraints, synthesis works best when it can see the whole path.  Register the outputs of the blocks that are synthesized independently, to solve this problem.

Note that registering the outputs of all blocks is not necessary.  For example if a chip is divided into four major hierarchical blocks which are synthesized independently, only the outputs of those four blocks need be registered.  Their sub-blocks do not require registered outputs because Design Compiler can see across the sub-block hierarchy during the compile.  The performance improvements of the 1998.02 release of Design Compiler allow for compiling larger blocks.  This makes the registered outputs methodology more practical to implement.

## Instantiate complex DesignWare parts

Although many of the fast DesignWare parts are inferred through simple HDL operators, several high performance parts require instantiation or a function call.  These are the MAC, SOP and vector adder.  Each of these parts can be found in the DesignWare-Foundation library.  They offer significant performance boost compared to their HDL coded equivalents.  Table 3 provides an example of their benefits using 16-bit inputs.

Table 3: High Performance DesignWare Parts – Normalized results (HDL equivalent code = 1)

|  | Part Number | Function | Delay | Area | Run-time |
| --- | --- | --- | --- | --- | --- |
| Multiply-Accumulate | DW02_mac | A*B+C | 0.85 | 0.91 | 1.03 |
| Sum-of-Products | DW02_prod_sum | A*B+C*D+… | 0.83 | 0.78 | 0.56 |
| Vector Adder | DW02_sum | A+B+C+D+… | 0.88 | 0.56 | 0.81 |

# Developing a Compile Strategy

This section discusses strategies for using the features of Design Compiler to produce a good initial gate-level implementation.  Section 5 then discusses how to analyze the synthesis results and make additional improvements.

There is no one script that produces the best results for all designs.  No synthesis tool (or human) can generate the best performance that is theoretically possible.  The goal should be to produce a netlist that meets functionality, constraints, and project schedule using a methodology that is as automated as possible.  The methodology outlined begins with a baseline approach and builds from there through successive experimentation and refinement.

## Top-down vs. bottom-up compiles

When synthesizing a hierarchical design one of the most important tasks is to decide which levels in the hierarchy are compiled.  One end of the spectrum is to compile the entire design top-down.  At the other end is a bottom-up compile approach that compiles every level, working from the bottom blocks to the top.  A top-down approach usually produces better results and always reduces the number of scripts the designer has to write.  However for very large designs run-time will become prohibitive.  Therefore the designer must pick sub-blocks, in the middle of the hierarchy, which to compile top-down, and then integrate them in a bottom-up fashion.

Another factor to consider is how to handle blocks that are instantiated many times.  A simple approach is to use the uniquify command, which will make copies of the block before the compile.  If the environment around each instance is very different, uniquify may produce better timing.  However run-time can be reduced significantly by compiling the block once and placing a dont_touch on it.  Then the design can be compiled top-down with less run-time.  Solvit article Synthesis-303 discusses this further.

### Compiling top-down

Whether you are designing a space station or a computer chip, problems always surface at the interfaces.  Bad sub-block constraints produce slower designs at the top.  By compiling the major sub-blocks (or the entire design) top-down, the number of interfaces is reduced.  During the compile Design Compiler can see across the hierarchy and accurately compute the delays, loads, fanout, etc.  This allows the tool to determine which side of a hierarchical crossing is the best for further optimization.  For most designs a top-down approach will produce better timing then a bottom-up flow.

In summary, there are several advantages of compiling top-down:

- Usually better timing results

- Fewer scripts to write and maintain

- The scripts are easier to understand and migrate to other designers and projects

### Compiling bottom-up

In a perfect world with infinitely fast tools and workstations, almost all designs would be compiled top-down.  Reducing run-time is the major motivator for compiling bottom up (often in parallel across multiple workstations).  The size of block that can be compiled top-down is not fixed; it is determined by the designers view of an acceptable run-time.  Often blocks of 100K gates can be compiled overnight using 1998.02 or later.  Since many factors affect run-time, the designer should experiment with blocks of various sizes.

Factors that increase run-time (in no order)

- Multiple clock domains

- Many timing exceptions (especially –through exceptions)

- Latch-based designs

- Tightly constrained designs

- Large amount of synthesized logic (gate count)

- Large register files

- Large critical range

- Many path groups

Based on experiments with actual RTL, a project can pick blocks, within the upper levels of hierarchy, to compile top-down.  These blocks are then integrated.  If any compiles occur with a mix of mapped and unmapped blocks, the mapped blocks are dont_touch’ed.  Later an incremental compile can be done with the dont_touch’s removed.

Although compiling large blocks is advantageous, compile smaller blocks during the initial phases of RTL coding.  This will allow for quick identification of the critical paths for possible recoding.  Once the RTL code stabilizes, perform compiles at higher levels of the hierarchy to generate the final netlist.

#### Creating design-budgets for bottom-up compiles

When performing a compile of a sub-block it is very important to use appropriate I/O delay constraints as well as accurate drive and load data.  This is a very difficult task to do manually.  For a path that begins in block A and ends in block B, the designer must estimate what percentage of the path exists in block A vs. block B.  Invariably certain I/O’s are over-constrained and others are under-constrained.   This directs Design Compiler to work on the wrong paths when performing timing-driven structuring and mapping.  Once the sub-blocks are integrated at the top, the real critical paths are discovered.  Then Design Compiler must work to repair the bad structuring and mapping done during the sub-block compiles.  The overall effect is increased run-time and worse performance.

The solution to this problem is automatic design-budgeting.  Starting with the 1998.08 release, Design Compiler can create sub-block constraint budgets based on the top-level constraints.  Design Budgeting will create constraints for input delays, output delays, drives, loads, fanout, timing exceptions, and more.

#### Design-budgeting vs. characterize

While Design Budgeting is a new feature, the characterize command has been around for many years.  Design Budgeting is invoked via the allocate_hierarchical_budgets command (use 
‘-levels 0’ to prevent recursive sub-block budgeting).  Both Design Budgeting and characterize take the constraints for the current design and create constraints for sub-blocks.  The type of constraints generated (e.g. I/O delays, drives, loads, timing exceptions, etc.) are the same for both techniques.

The difference between allocate_hierarchical_budgets and characterize lies in how the I/O delay constraints are created.  For example, to determine the constraint for a sub-block input pin, characterize analyzes all paths leading to the pin and picks the longest delay.  A similar analysis is done for all sub-block output pins.  characterize works with one sub-block at a time.  allocate_hierarchical_budgets on the other hand works with multiple sub-blocks at once.  It analyzes all paths that go from one sub-block to another and determines I/O delay constraints such that each block is given the best opportunity to meet timing.

The technical details of two commands is unimportant if the designer knows when to use each.  allocate_hierarchical_budgets is used to generate constraints for sub-blocks that are compiled in parallel.  Ideally if each sub-block meets its budgeted constraints then the entire design will meet it’s top-level constraints (certain interface effects are very difficult to model and often result in the need to perform an incremental compile at the top).  characterize is used when the design is already mapped to gates and the designer wants to perform a focused compile on one sub-block.  characterize should not be used to compile designs in parallel; it will result in over and under constraining.

Like characterize, budgeting works off of a gate-level design.  To generate the first set of budgets, each block to be budgeted is compiled by the user with only drive and load constraints (this results in a quick mapping).  Whenever budgets are created, add some margin to the I/O timing constraints.  This can be specified via the set_context_margin command before budgeting.  A project should plan to run Design Budgeting several times.  Each loop through budgeting, compiling and budgeting will refine the budgets.  See the ‘Design Compiler Reference Manual – Timing & Constraints’ for details on the flow.

#### The Do’s and Don’ts of dont_touch’s

A common bottom-up compile strategy is to individually compile sub-blocks, moving up the hierarchy, then perform an incremental compile at the top.  Before the incremental compile some designers place a dont_touch on all of the sub-blocks.  This can have a negative effect on circuit performance.  The idea is to force Design Compiler to only buffer the sub-block interconnects.  However it is quite possible that further improvements could be done if Design Compiler were allowed to optimize logic within the sub-blocks.  Therefore it is recommended that dont_touch’s NOT be placed on sub-blocks when performing an incremental compile.

One motivation for placing dont_touch’s on sub-blocks is to reduce run-time.  There are better strategies for top-level fix-up compiles.  These are discussed in section 4.9.

## Baseline compile approach

The core of an effective compile strategy is a medium effort compile followed by a high effort incremental compile.  The medium effort compile will finish in a reasonable run-time, allowing analysis of the results while the high effort incremental compile runs in parallel.  Note that the effort level does not tell Design Compiler to just work longer.  Different timing and area optimization algorithms are added by each successive effort level (low, medium, high).  A low effort compile should be avoided for high performance designs because it restricts itself to area based resource sharing and implementation selection.

Reports should be generated after the compile, for analysis as discussed in Section 5.1.   After the baseline compile is completed, other techniques can be attempted.  Techniques that show improvement should be retained.

Example Baseline Compile

- include start_time.scr -quiet

compile -map_effort medium

write -hier -out my_design.initcomp.db

report_timing -nets -trans -sig 3

report_area

report_constraints

compile -incremental -map_effort high

write -hier -out my_design.inccomp.db

report_timing -nets -trans -sig 3

report_area

report_constraints

include end_time.scr -quiet

The start_time.scr and end_time.scr scripts record the start and end wall-clock and CPU times, and print useful run-time statistics.  The source code can be found in Solvit article Synthesis-291.

## Using appropriate wire-load models

Many ASIC vendor libraries allow Design Compiler to automatically pick wire-load models (WLMs) based on each block’s area.  There are cases where the designer will want to override this.  If the RTL code contains very small blocks, Design Compiler will pick a very small WLM from the library.  Often in the floorplan and layout, these small blocks are ungrouped.  This makes Design Compiler’s selection optimistic, resulting in more violations after back-annotation.  The ‘set_wire_load –min_block_size’ command is used to designate the smallest WLM for Design Compiler to use.

Custom WLMs can be important for meeting timing on high performance designs.  Custom WLMs are created using parasitic data from the floorplan or layout.  Once they are created, synthesis of the RTL is re-run.  This results in more accurate synthesis and fewer iterations through the back end.  Custom WLMs can be created using Floorplan Manager.  See the Floorplan Manager Users Guide for details.

## Using a critical range

During a compile Design Compiler continuously works to improve the paths leading to the most critical endpoint (e.g. an output port or flip-flop).  The endpoint which is most critical will change from moment to moment as paths are improved.  Once Design Compiler determines it can not further improve the most critical endpoint it exits the delay optimization phase and moves into design rule fixing.

If a critical range is used, Design Compiler will work on more endpoints than just the most critical one.  Therefore if the most critical endpoint can not be further improved, Design Compiler will move on to the next most critical.  The critical range is set via the set_critical_range command (before 1998.02 this was compile_default_critical_range) whose argument is time (often in nano-seconds).  If a critical range of 2ns is specified, Design Compiler will work on optimizing the most critical endpoint and endpoints within 2ns of the most critical endpoint.  Note that critical range does not affect the initial mapping; the RTL is mapped to the target technology even if no critical range is set.  Also, even with a critical range Design Compiler will not work to improve paths that are already passing their timing constraint (to add margin, increase the -minus_uncertainty via the set_clock_skew command).

Using a critical range will often improve the worst violator in the design.  A very large critical range (e.g. 1000ns) will cause Design Compiler to work on all failing endpoints in the design.  This in turn will cause an undesirable increase in run-time.  The goal is to improve the worst violator with minimum increase in run-time.  Picking a small value for the critical range does this.  One rule of thumb is to use a value that is around one cell delay in your technology (e.g. 250ps).  If the design is already mapped to gates another rule of thumb is to run a ‘report_timing -path end -max_paths 20’ and pick a critical range that will force Design Compiler to work on all the endpoints shown.  The number 20 is a general guideline meant to keep run-time low.

Another reason to use a critical range is to force Design Compiler to work on all failing endpoints before handing off to layout.  Even though the design may not meet the timing goal, the number of failing paths will be reduced (i.e. the Total Negative Slack is reduced).  This can be done by setting the critical range to any large number (say 1000ns). See Section 5.1 for further discussion of using critical range.

## Impact of set_max_area on timing

Starting in 1998.02 Design Compiler will perform minimal area optimization unless an area constraint is set.  Previous versions of Design Compiler always optimized for area if timing constraints were not met.  During area optimization Design Compiler maintains delay as a higher priority then area.  Area optimization sometimes uncovers opportunities for Design Compiler to make further timing improvements.

Try compiling with and without a set_max_area constraint.  If the timing does not improve with it then consider forgoing the area constraint until later in the design cycle.  Early in the design cycle the priority is delay and getting results quickly; an area constraint will increase the run-time.  If a realistic area constraint is given, Design Compiler will quit the area optimization phase when it is reached.  A constraint of zero can be used, but Design Compiler will work longer before finally giving up.

## Using boundary optimization

There are four types of optimizations which are collectively referred to as boundary optimization:

- Propagation of constants across hierarchy

- Propagation of Equal/Opposite information across hierarchy

- Propagation of unconnected port information across hierarchy

- Pushing inversions across hierarchy

This type of data is always propagated within a block but normally does not cross into sub-blocks.  When boundary optimization is enabled, all four types occur up and down the hierarchy.  This can result in a faster critical path and smaller design.  Enabling boundary optimization does not allow Design Compiler to share logic across the hierarchy; this can only happen if the hierarchy is ungrouped.

By defaul, the compile command will propagate constants and equal/opposite information up (not down) the hierarchy.  If the compile_preserve_subdesign_interfaces variable is set to TRUE even this basic level of propagation does not occur.  However if boundary optimization is enabled, the compile_preserve_subdesign_interfaces variable is ignored.

Characterize can activate a limited form of boundary optimization.  If ‘characterize 
-connections’ is used, the attributes for the first three forms of boundary optimization are applied to the inputs of the sub-block.  These attributes are propagated into the sub-block during any subsequent compile of that block, but will stop at any hierarchy below it.

Boundary optimization is enabled via the -boundary_optimization switch to the compile command.  Alternatively the set_boundary_optimization attribute can be set to true or false on individual designs, to invoke it during compile.

### Issues with boundary optimization

Although boundary optimization can improve compile results, there are several issues with using it that must be understood. A specific type of transform possible with boundary optimization is the elimination of inverter pairs separated by hierarchy.  This type of transform reduces delay on the path but changes the phase of the I/O port.  To represent this, Design Compiler adds ‘_BAR’ to the port name.  If gate-level simulation of the sub-block is performed the _BAR suffix may necessitate tweaks to the testbench.

This inverter pushing optimization also effects formal verification (FV).  Equivalence checking FV tools compare two versions of the same design for functional equivalence.  When performing a gates-to-gates check, the FV tool will try to match all I/O ports across the two designs.  Boundary optimization can change the phase and name of sub-block ports.  This creates a mismatch for the FV tool.  This is not an issue if FV can be done from the top-level (the top-level functionality is unchanged by boundary optimization).  If run-time dictates that FV be performed at the sub-block level, one strategy is to use the set_boundary_optimization command to activate boundary optimization only on the sub-blocks below the blocks that are compared with FV.   If this command is used to select specific blocks, remove the        
-boundary_optimization switch from the compile command.  A simpler work-around is to set compile_disable_hierarchical_inverter_opt = true.  This will disable the inverter pushing algorithm of boundary optimization.

Boundary optimization can change the functionality of a sub-block.  For example, consider two sub-block inputs that are ANDed together and fed to a flip-flop.  If both inputs are tied high at the parent block level, the AND gate is not needed and the flip-flop data input is tied high.  The top-level functionality has not been changed, however the function of the sub-block itself has changed.  By itself this is generally not an issue unless gate-level simulation is performed on the sub-block.  In that case, gate-level simulation results will differ from RTL simulation.  However, the top-level simulation results are the same.

Another issue involves recompiling of sub-blocks from RTL.  Constants within a sub-block will propagate out of the block and remove logic in the parent block.  Once again the top-level functionality is not affected, and the design becomes smaller and faster.  However, problems can occur if the sub-block RTL is modified, recompiled, and slipped into the existing parent design.  The constants that previously propagated out of the sub-block may have been removed by the RTL modifications.  Now the sub-block may be expecting certain logic within the parent block that has been removed by the previous boundary optimization compile.  The result is that the top-level functionality is changed.  If this flow is to be used, do not use boundary optimization, and set the compile_preserve_subdesign_interfaces variable to TRUE.

## Using SOP Flattening

Sum-of-Products (SOP) flattening is an optimization technique used early during a full compile (i.e. it is not performed during an incremental compile).  SOP flattening involves converting the equations of a logic cone to a sum-of-products expression (e.g. y=abc+def+ae).  A SOP form usually consists of only two or three levels of logic (depending on inversions and fanin of available gates).  If after compiling, the critical path has 10 levels of logic when three are needed, try SOP flattening; no other technique will provide such a reduction in levels of logic.  SOP flattening is turned off by default in Design Compiler.  This is because certain types of structures experience gate-count explosion when flattened.

Table 4: Which blocks respond well to SOP flattening?

| Good candidates for SOP flattening | Bad candidates for SOP flattening |
| --- | --- |
| State machines | Arithmetic components |
| Random logic | XOR structures (parity trees) |
| Blocks that use don’t-cares | Blocks with many muxes |

A standard approach for using SOP flattening is to first compile the design without it.  Then search the RTL of blocks along the critical paths.  Look for logic that is a good candidate.  Try flattening on these blocks and compare the results.  For some blocks flattening actually will increase the delay.  This is because flattening increases the fanout of the block inputs.  As such, loading of these nets will increase and can offset any gains derived from decreased levels of logic.  In general flattening may not work well for blocks that have a lot of outputs where each input is related to many of the outputs.

The set_flatten command enables SOP flattening.  One argument to set_flatten is
-phase.  By default when Design Compiler flattens logic it creates K-maps and tries to cover the ‘1s’ in the K-map.  For K-maps with many 1s this will produce more logic then necessary.  Using 
-phase instructs Design Compiler to also consider covering the ‘0s’ of the K-map with the output inverted.  When -phase is used both options are considered, therefore -phase should always be used.

There are three effort levels for the set_flatten command.  Flattening will cause increased gate-count for certain structures.  The effort level determines how much of an area increase is allowed for each endpoint.  Low effort flattening will probably produce similar results as a default compile, therefore medium and high flattening efforts should be used.  If flattening is attempted on a block with good and bad candidates, Design Compiler will give up on the bad candidate endpoints and keep their original logic structure.  Only use SOP flattening on blocks which show a timing improvement; this will help keep the area impact down.

Another option to set_flatten is  ‘-minimize <single_output | multiple_output>‘.  This instructs Design Compiler whether to flatten each endpoint independently or to share logic across endpoints.  With single_output, Design Compiler will minimize the equations for each output individually. This results in the smallest implementation for each output, but the design as a whole might suffer because product (AND) terms are not shared well between outputs.  With multiple_output Design Compiler will share as many product (AND) terms between outputs as possible. This strategy is beneficial for ROM-like structures in which most outputs are a function of all inputs.  In the final analysis it is difficult to determine which method will work best.  Experiment with both methods for a given block.

Example of using SOP flattening

set_flatten true -phase true -minimize single_output \

-design {blockA blockB} -effort medium

set_flatten true -phase true -minimize multiple_output \

-design {blockC} -effort medium

compile

## Use the fast DesignWare parts

Many DesignWare parts are included with a Design Compiler license.  These reside in the standard.sldb library.  The Foundation Library provides higher performing implementations of the standard parts as well as many additional functions.  These include a fast CLA adder (CLF), Wallace tree multiplier, CLA comparators, and a vector adder.  Many of the most popular Foundation Library parts are inferred through the RTL code using operators such as ‘+’ and ‘*’.

Even if a Foundation Library license exists, the library must be activated in order for Design Compiler to use its faster parts.  This is done by adding the library files to the synthetic_library and link_library variables:

synthetic_library = {dw01.sldb, dw02.sldb}

link_library = link_library + synthetic_library

Without these commands, Design Compiler will always pick the slower implementations found in the standard library.  Note the link_library addition is only needed if DesignWare parts are instantiated in the design.  A convenient location for these variables is in the .synopsys_dc.setup file.

The DesignWare Foundation library is updated four times a year.  Therefore even though a project is using the latest version of Design Compiler, it may be a release or two behind for DesignWare.  This is important because new releases sometimes contain faster implementations of popular parts.  Details about the latest release and download instructions can be received by sending a blank email to dw_est@synopsys.com.

## Fast Top-Level Incremental Compiles

As discussed in Section 4.1.1 bottom-up compiles often suffer from bad sub-block constraints.  Once the sub-blocks are integrated at the top, interface timing problems are often found.  A medium or high effort incremental compile is the best initial approach to take.  However if the design is very large the run-time of a normal incremental compile may be prohibitive.  There are several strategies for improving run-time in these situations.

If a bottom-up compile strategy was used, a top-level incremental compile should only be used to fix violations between budgeted blocks.  If the only violations exist on register-to-register paths within a budgeted block, tackle the problem there.  Design Budgeting will minimize the number of timing violations between budgeted blocks.

### Recommended flow to reduce run-time of incremental compiles

The standard two-pass synthesis strategy calls for a medium effort compile followed by a high effort incremental compile.  Run-time for the high effort incremental compile can be reduced in several ways.

First recall that any critical range will increase run-time.  Therefore pick a small value or zero (See Section 4.4).  Next turn off area optimization by removing any max_area attribute that was previously set.  Another technique is to disable automatic wire-load selection.  Throughout synthesis, Design Compiler re-evaluates all of the wire-load models and changes them if a design’s area has changed to warrant a new model.  However if the designer is satisfied that the current wire-load models are correct, automatic wire-load selection can be disabled resulting in less run-time.  The default_wire_load attribute must be removed from all libraries to prevent Design Compiler from using the default.  This technique works best for designs with many levels of hierarchy.  Later, automatic wire-load selection can be enabled and static-timing analysis run.

Finally, incremental implementation selection (IIS) of DesignWare parts can be turned off.  IIS visits each DesignWare part and evaluates changing its implementation (e.g. RPL vs. CLA).  If the designer is satisfied that Design Compiler is using the best DesignWare parts it can, IIS can be turned off to improve run-time.

Improving run-time of top-level incremental compiles

set_critical_range 0.0 current_design 
remove_attribute current_design max_area
auto_wire_load_selection = false
remove_attribute find(library) default_wire_load
compile_implementation_selection = false
compile -incremental -map_effort medium

### Dont_Touch’s on Sub-blocks

A simple approach for improving run-time of top-level compiles is to place a dont_touch on the top-level sub-block instances.

The downside to this approach is that only interconnect logic is optimized (cells are sized and buffers added/removed).  Further optimizations could very likely be done if access to the sub-blocks were allowed.  Although this strategy may be successful in some cases, it is generally recommended that dont_touch’s NOT be placed on sub-blocks.

Dont_Touch’s can be placed on the top-level blocks using the following dc_shell script:

filter( find(cell),"is_hierarchical == true" )
foreach(tmp, dc_shell_status) {
  set_dont_touch tmp
}

This approach will improve run-time but keep in mind that the sub-block logic is still there.  Timing analysis is still done as changes are made to the netlist, so the compile will not be instantaneous.

### Variables for pre-1998.02 Design Compiler

This section discusses several variables, which improve run-time for pre-1998.02 releases of Design Compiler.  They should be used in addition to the techniques highlighted in the rest of Section 4.9.  These variables are not needed for 1998.02 and beyond, and can hinder results.   Therefore they should be removed from scripts when using 1998.02 or later.

The 1998.02 version of Design Compiler is 3-5x faster then previous releases.  However it is quite possible that some projects are near tape-out and would like to avoid upgrading.  These projects can use several variables to improve run-time, possibly at the expense of the quality of results (note that 1998.02 will still be much faster).

Pre-1998.02 compiles devote much time to downsizing cells even when no area constraint is set.  Using the compile_restrict_down_sizing variable (set to true) will disable all downsizing.  Note there is a related variable, compile_limit_down_sizing (set to true), that will only reduce the amount of downsizing performed.  The compile_use_fast_sequential_mode variable instructs Design Compiler to only perform incremental sequential optimizations on sequential components on the critical path (as opposed to all sequential components in the design).

If either of the down sizing variables are used, the design area will be larger.  This can be remedied by following up the first incremental compile with a low effort incremental compile with the down sizing variables set to false.  The low effort incremental compile will not devote much run-time to timing optimization, but will perform area reduction.

How to Improve Run-Time for Pre-1998.02 Releases

compile_restrict_down_sizing = true
compile_use_fast_sequential_mode = true
compile –incremental –map_effort high
compile_restrict_down_sizing = false
compile –incremental –map_effort low

## DC Ultra

The DC Ultra license adds additional capabilities to Design Compiler for high performance designs.  This includes BOA and BRT, as described in Section 4.11.  One feature of DC Ultra is a set of algorithms that improve circuit performance during compile.  These algorithms are enabled via the set_ultra_mode command and invoked during a high effort compile (full compile or incremental compile). set_ultra_mode allows Design Compiler to duplicate whole fanin cones to reduce the loading of critical paths.  It also maps to more complex and wide fanin gates.  In benchmark studies, DC Ultra improved circuit performance over DC Expert for roughly 50% of designs tested and by an average of 10%.  If a license is available, experiment to determine which blocks respond well to a high effort incremental compile using set_ultra_mode.

## Using BOA and BRT

BOA (Behavioral Optimization of Arithmetic) is a feature that applies various optimizations on arithmetic expressions.  It is invoked via the transform_csa command, which is available with either DC-Ultra or Behavioral Compiler (previous to 1998.08, it was only available with Behavioral Compiler).  BOA performs optimizations that increase the performance of datapath designs.  Examples of where BOA works well are datapath intensive designs (contain *,+,- operations), arithmetic operations of 8 bits or larger, and any multiplications by a constant.  BOA replaces complex arithmetic expressions with a carry-save array tree.  It also performs canonical encoding which can greatly improve the performance of constant multiplications.  In addition it can decompose multiplications into a series of shifts and adds.  All of these optimizations are performed automatically using the transform_csa command.

BOA often makes a datapath 10-20% faster and 0-15% smaller.  Some of the optimizations done by BOA are similar to those found in the SOP, MAC, and Vector Adder DesignWare parts.  However BOA also works on much more complex arithmetic expressions and does not require modifications to the source code.  BOA is invoked after elaboration and before the compile command.  Details on using BOA can be found in the BOA User Guide available via the on-line docs (iview).

BRT (Behavioral Re-Timing) is a technique for moving registers across combinational gates in order to improve timing and minimize sequential area.  It is available with either DC-Ultra or Behavioral Compiler (previous to 1998.08, it was only available with Behavioral Compiler).  BRT consists of two commands, pipeline_design and optimize_registers. pipeline_design is used to pipeline a purely combinational block.  The designer does not have to decide where in their RTL to place the registers for optimal performance.  The desired function is coded as a large combinational block and pipeline_design adds pipe stages and places them to achieve maximum performance.  optimize_registers moves registers in an existing sequential design to improve performance.  It performs well on designs which do not contain asynchronous resets or enabled flip-flops.

BOA and BRT make a powerful combination for datapath blocks.  BOA optimizes the arithmetic expressions, then BRT adds pipe stages.  The designer benefits from minimal coding effort.  The equivalent manually generated RTL code is often many pages long and very low level, making it difficult to debug.  Details on using BRT can be found in the BRT Users Guide via the on-line docs.

A feature similar to BRT, balance_registers, is built into Design Compiler.  The resulting timing may not be as good as BRT, and it tends to increase area.  Also, BRT can optimize across hierarchy.  See its man-page for details.

# Post-compile Analysis and Re-optimization

After performing the baseline compile of a design there still exist many opportunities for improvements.  These efforts focus on the critical paths in the design.  The following sections discuss how to analyze the results of a compile and a variety of methods for making further improvements.  If many violations exist try large-scale techniques such as modifying the RTL code or re-compiling sub-blocks.  If only a few violations exist they might be fixed using tactical approaches such as cell sizing or re-buffering.

## How bad is it? – Gathering metrics

Several metrics should be gathered to analyze the quality of the synthesized design.  These metrics are used to determine the critical paths of the design, bottlenecks, and the distribution of the path slack.  The following script offers an example of how to collect design metrics.

Gathering Post-Compile Quality Metrics

include start_time.scr –quiet

<compile_the_design>

include end_time.scr -quiet

report_constraint

report_area

report_timing –nets –trans –sig 3

report_timing –path end –max_paths 50

report_timing –nets –trans –sig 3 –max_paths 100 > somefile

include find_dw_implementations.scr –quiet > someotherfile

The find_dw_implementations.scr script reports the type of DesignWare parts used throughout the hierarchy.  It can be found in Solvit article Synthesis-312.

To investigate a timing violation, run a timing report using ‘-net -trans -sig 3’ and closely examine it.  Sometimes a cell is found that has a much larger delay than other cells along the path.  Look to see if the transition time (provided by -trans) of either the input or output net is large.  If so the net could be overloaded.  Nets with a high load can slow the driver cell, load cell (from slow transition times) or both.  Look at the fanout on each net (provided by -net) to confirm.  When a case like this is found it may be beneficial to manually upsize a cell using change_link or buffer the net using balance_buffer.  Both are described in Section 5.5.  If the slow transition time is at the output of a flip-flop, check the flip-flop’s clock input transition time.  If it is very high then a clock buffer probably has not been instantiated or the clock net drive strength has been understated.  The –sig switch  increases the significant digits of the displayed values.  This is useful for reading values as pico-seconds.

‘report_timing -path end’displays a one line summary for the critical path.  Combined with

‘-max_paths’, report_timing produces a textual slack distribution of the design.  The distribution can be viewed graphically using Primetime.  Examining the slack distribution can give an indication of how good or bad the overall timing is.  The designer may find that almost all of the failing paths are clustered around the worst negative slack.  This could indicate that only one or a few bottlenecks are causing most of the violations.  Or it may be found that many paths are failing and the distribution is rather uniform.  This could indicate that many areas of the design require improvement.

Examining the slack distribution is useful, however it is misleading if a critical range was not used during the compile.  Without a critical range Design Compiler only works on the worst violator in each path group.  If a critical range was not set it should be expected that many paths will be violating (because Design Compiler was focusing only on the worst path).  Therefore the slack distribution is useless outside of the critical range.  The paths Design Compiler worked on lie within the critical range.  These are the paths the designer should examine closely.

Positive Slack

# Paths

Critical Range

Negative Slack

Positive Slack

Negative Slack

# Paths

Figure 1a:						Figure 1b:				
Slack Distribution – No Critical Range			Slack Distribution – With Critical Range

Table 5 shows a summary of various types of analysis which can be done using report_timing.

Table 5: Metric reporting summary

| Report | Info displayed | Possible problems | What to do next |
| --- | --- | --- | --- |
| any report_timing | - Critical Path | - Path crosses many levels of hierarchy. | - Ungroup hierarchy and do an incr. compile. |
| report_timing –nets | - Fanout count for each net | - A cell delay much larger than others. - A net with large fanout. | - Size cells using change_link. - Buffer the net using balance_buffer. |
| report_timing -trans | - Transition time of each cell output | - A cell delay much larger than others. - A cell transition time much larger than others. | - Verify clocks are driven by large drive strength. - Buffer the net using balance_buffer. - Size cells using change_link. |
| report_timing –path end –max_paths 50 | - One line summary for first 50 endpoints (the slack distribution). | - Many violations within the critical range. | - Perform focused compile of bottleneck blocks (Section 5.3). - Recode bottleneck RTL. |
| report_timing –max_paths 100 | - Paths for the 100 most critical endpoints. | - Many failing paths go through the same few blocks. | - Perform focused compile of bottleneck blocks (Section 5.3). - Recode bottleneck RTL. |

## Bottleneck analysis

Slow sections of a design can create bottlenecks.  These are sub-blocks, functional operations, or individual cells which appear in many of the failing paths.  Once identified, bottlenecks represent opportunities to improve many paths at once.  Examine the ‘-max_paths 100’ timing report for sub-blocks, functional operations or leaf cells which appear in many paths.  Primetime can be used to graphically perform this analysis.

Bottleneck sub-blocks or DesignWare parts are candidates for the characterize/compile strategy as discussed in Section 5.3.  Sections of bottleneck random logic are candidates for SOP flattening as discussed in Section 4.7, or RTL recoding as discussed in Section 1.  Individual bottleneck cells many be caused by implicit dont_touch’s as discussed in Section 5.6 or may benefit from buffering as discussed in Section 5.5

## Re-optimization of sub-blocks

The timing of a sub-block can often be improved by performing a compile of it using updated constraints.  The compile may be a high effort incremental compile, or if necessary a full compile from RTL followed by an incremental compile.

Design Budgeting or characterization can create the updated constraints.  Design Budgeting should be used when multiple sub-blocks are to be re-optimized in parallel.  Design Budgeting will find constraint budgets that will give each block the best change of meeting its constraints.  Characterization should be used when the designer wants to re-optimize one or more blocks in series (e.g. characterize block A, re-optimize block A, characterize block B, re-optimize block B, etc.)  Don’t compile blocks in parallel using characterized constraints.

When creating constraints for sub-blocks, first set the current design to the highest level possible.  Design Compiler uses the current design’s constraints as its basis.  Thus it is important to create the sub-block constraints from a level where the constraints are trusted; many times this is the top-level of the ASIC.  If accurate design budgeting has been done on major sub-blocks, constraints can be generated from there.  For example, if sub-block instance u100 is several levels down in the hierarchy, the characterize command might look like this: ‘characterize -constraints -connections {instA/instB/instC/u100}’.

When compiling a sub-block, interface effects exist which must be addressed.  During a compile, Design Compiler might increase the load on an input port.  Design Compiler is able to compute the effect of the increased load on the block being compiled, however it has no way to determine its affect on other blocks which share the same net driving the input.  Increasing the load on an input port may cause a timing violation in another block.  Another issue occurs at the sub-block outputs.  It is possible that during a sub-block compile Design Compiler will make an output transition time worse.  Once again Design Compiler will ensure that this does not cause a violation within the block being compiled.  However the delay of the next cell downstream (outside the block) is made worse.  Therefore the block being compiled may meet timing but the overall timing will fail once top-level analysis is performed.  These effects are very difficult to model.

To address the issues described above, add margin to the sub-block I/O constraints.  Increasing each set_input_delay and set_output_delay value by a small amount does this.  A good value for the adjustment is a typical cell delay (e.g. 250ps).  If Design Budgeting is being used to create the sub-block constraints, use the set_context_margin command.  If characterize is being used, a script named tighten_constraints.scr exists which does this automatically.  It can be found in Solvit article Methodology-48.

### Optimization with updated design budgets

After a bottom-up compile, timing violations might be seen within a budgeted sub-block, across budgeted sub-blocks, or both.  If none of the violations cross budgeted sub-blocks, they can be addressed via further optimization within the sub-block, with no need to update the budgets.  If violations do occur across budgeted sub-blocks, the budgets should be updated.

The budgets are updated by running the allocate_hierarchical_budgets command using the existing gate-level design (again use ‘-levels 0’ if budgets are only desired for top-level blocks).  After updating the budgets, incremental compiles can be done in parallel, of the blocks which have violations.  If the violations across the budgeted sub-blocks are too severe for an incremental compile to fix, consider a full compile from RTL.  This will allow restructuring of the logic using the updated constraints.

### Optimization with characterized constraints

Characterization should be used when the designer wants to re-optimize one or more blocks in series.  Do not compile blocks in parallel using characterized constraints; I/O paths will be over and under constrained resulting in inferior timing.  When using characterize, most designers will want to use the -constraints  switch.  It specifies that Design Compiler is to add design rule constraints (e.g. set_max_fanout) that were set on the top block.  In addition it is often useful to include the 
-connections switch.  This adds connection information to the sub-block constraints.  For example, if an input to the sub-block is tied low, this switch will add a set_logic_zero to that port.  Design Compiler can use this information to remove unneeded logic.  In this way -connections can modify the functionality of a sub-block just as boundary optimization does.  The top-level functionality remains the same.

#### Characterizing sub-blocks

When following the characterize/compile strategy, the sub-blocks should be compiled in series.  For example block A is characterized and recompiled, then block B is characterized and recompiled, etc.  The following example characterizes two sub-blocks (designA and designB) and re-compiles each.  The tighten_constraints.scr script is used to tighten the I/O constraints of the sub-block before the compile.

Example sub-block characterize/compile flow

characterize -constraints -connections {instA}

current_design designA

include tighten_constraints.scr -quiet

compile -inc -boundary_opt -map_effort high

current_design TOP_DESIGN

report_timing -net -trans -sig 3

characterize -constraints -connections {instB}

current_design designB

include tighten_constraints.scr -quiet

compile -inc -boundary_opt -map_effort high

current_design TOP_DESIGN

report_timing -net -trans -sig 3

After characterizing and compiling a block, return to the top-level and perform timing analysis.  Sometimes the top-level timing analysis will look worse then before the characterize/compile.  If this occurs perform a top level medium or high effort incremental compile.  Only then will the success of the characterize/compile be seen.

#### Characterizing DesignWare parts

All of the comments of Section 5.3.2.1 apply for DesignWare parts, with the addition of one step.  The part could be recompiled just like any other sub-block, however the potential for Incremental Implementation Selection (IIS) would be lost.  Design Compiler re-evaluates the implementation (e.g. RPL vs. CLA) of DesignWare parts during a compile or incremental compile.  By maintaining the DesignWare part as an instance within a parent block, Design Compiler makes it a candidate for IIS.  Placing the DesignWare part in a dummy level of hierarchy does this.

The following example recompiles a DesignWare part, instance u100, which resides in block TOP_DESIGN/blockA.  The instance name for blockA is blockA_0.  u100 is placed in a dummy level of hierarchy, this dummy level is then characterized, the I/O constraints are tightened and a high effort compile is done.  Note that a full compile (not an incremental) is performed.  This allows Design Compiler to restructure the DesignWare part from scratch, using the accurate timing constraints provided by characterize.

Example DesignWare characterize/compile flow

current_design blockA

group u100 -design dw_dummy -cell dw_dummy0

current_design TOP_DESIGN

characterize -constraints -connections {blockA_0/dw_dummy0}

current_design dw_dummy

include tighten_constraints.scr -quiet

compile -boundary_opt -map_effort high

current_design TOP_DESIGN

report_timing -net -trans -sig 3

## Ungroup hierarchy

Design Compiler looks for opportunities for merging discrete cells into complex ones.  When a path crosses a hierarchical boundary, Design Compiler can not merge cells on opposite sides of the boundary.  This can result in more levels of logic and a longer delay on the path.

Examine the critical path for hierarchical crossings.  If it passes through numerous small blocks, ungroup these blocks and do a high effort incremental compile.  Also ungroup any DesignWare parts which lie on the critical path.  Any path within the critical range can benefit from this technique.  Many floorplan and layout tools automatically ungroup all hierarchy smaller than a certain size.  This hierarchy is a prime candidate for ungrouping.

Ungrouping hierarchy can improve the critical paths, but it can make them worse just as easily.  Paths can get worse when the sub-block that was ungrouped used a smaller wire-load model than its parent block.  This increases the load on each of the nets that were fully contained within the sub-block.  The designer must determine which wire-load model is most accurate.  If the sub-block is going to be ungrouped at the back-end anyway, the smaller wire-load model may be too optimistic.  For example, if the entire design is to be ungrouped in layout, the best results might be obtained by ungrouping all hierarchy within Design Compiler.  However this is tricky and several iterations of custom wire-load model creation may be required to find a wire-load model that accurately reflects the routing.

## Improving high fanout nodes

The -nets switch on report_timing displays the fanout of each cell along the path.  If nets are found which have a high fanout, the net may benefit from a buffer tree.  Design Compiler can automatically add a structurally balanced tree of buffers via the balance_buffer command.  Note that this tree is not guaranteed to be delay balanced; there may be skew between each branch.  Therefore balance_buffer is not suited for clock tree synthesis.

To buffer the net n2 invoke ‘balance_buffer -net n2’.  The balance_buffer command has the limitation of only working within the current design.   If the high fanout net crosses hierarchy the designer may have to run balance_buffer several times or ungroup some of the hierarchy.

If many high fanout nets exist, an automated method is useful.  Solvit article SYNTH-480902 contains a combination Perl/dc_shell script that will run balance_buffer on all nets with fanout greater then a specified value.

While adding a buffer tree can reduce the delay of a high fanout net, a simpler technique is to manually size the driver cell.  This is done via the change_link command.  However before a cell is manually upsized, the designer should investigate why Design Compiler did not upsize the cell.  The reason is often an implicit dont_touch as discussed in Section 5.6.

## Implicit dont_touch’s

If a cell is found which has an abnormally high delay, check to see if the highest drive strength is being used.  If not, the reason is often an implicit dont_touch.  An implicit dont_touch is a dont_touch that was not explicitly placed on an object (via the set_dont_touch or set_attribute commands).  A dont_touch (whether implicit or explicit) on a cell will prevent sizing of the cell.  There are several causes of implicit dont_touch’s.

Table 6: Implicit dont_touch’s

| Causes of Implicit dont_touch’s | Comments |
| --- | --- |
| set_disable_timing | A set_disable_timing on a cell puts a dont_touch on the cell |
| -through timing exceptions | A –through on a cell’s pin puts a dont_touch on the cell |
| create_clock | A create_clock on a cell’s pin puts a dont_touch on the cell |
| set_dont_touch_network | Places a dont_touch on all nets and mapped combinational cells within the fanout cone of the set_dont_touch_network object |
| set_dont_touch on a net | Places a dont_touch on all mapped cells connected to the net |
| combinational feedback loops | Design Compiler automatically breaks the loop similar to doing a set_disable_timing on a cell; this places a dont_touch on the cell |
| path segmentation (set_input_delay or set_output_delay on a cell’s pin). | Places a dont_touch on the cell. Largely an obsolete technique since the release of 1998.02’s –through switch. |

An explicit dont_touch can be confirmed by performing a get_attribute on the object (e.g. ‘get_attribute u100 dont_touch’).  To find implicit dont_touch’s run a report_cell or report_net on the object.  To find the cause, redirect a write_script report to a file and search for ‘dont’.  An implicit dont_touch can not be removed via remove_attribute; to remove the implicit dont_touch, remove the cause.

## Look at the DesignWare implementations selected

Design Compiler evaluates DesignWare implementations during full and incremental compiles.  Even so a report should be run to identify the implementations used for DesignWare parts along the failing paths.  This is done via the report_resources command.  To report this data throughout the hierarchy, use the find_dw_implementations.scr script found in Solvit article Synthesis-312.

If a RPL adder or CSA multiplier is found, verify that the faster DesignWare components are enabled as discussed in Section 4.8.  Also be sure that good selections were made before ungrouping any DesignWare components.  set_implementation will force Design Compiler to use the specified implementation during subsequent compiles (full or incremental).

## Direct where Design Compiler focuses

Sometimes the critical paths of a design fall along the top-level I/O’s.  When this occurs the I/O delay constraints should be re-examined to verify they are realistic.  I/O constraints are often developed early in a project and may end up being highly unrealistic.  If this is the case Design Compiler will focus on these paths and not attempt to improve the register-to-register paths.  If the I/O constraints are unrealistic, the solution is to re-negotiate the constraints with the owner of the interfacing blocks or ASIC’s.  However in the meantime the designer may wish to direct Design Compiler to focus on the register-to-register paths.  Creating new path groups for the I/O paths will do this.

group_path -name in_paths  -critical_range 0.0 -from all_inputs()

group_path -name out_paths -critical_range 0.0 -to   all_outputs()

By default a path group exists for each clock domain.  Creating separate path groups for the I/O’s leaves the register-to-register paths in the original path group(s).  Since Design Compiler attempts to optimize the worst violator of each path group it will now work on the register-to-register paths in addition to the I/O’s.  The -critical_range  switch is optional.  Here it is being used to override any global critical range that has been set.  This will direct Design Compiler to devote less time to the I/O paths.

## Update the sub-block drive and loading constraints

If Design Budgeting is not being used, it is difficult and time consuming to update the time budgets of sub-blocks.   However it is still easy to update the sub-block drive and load constraints.  This can be done with the characterize command.  First each sub-block is characterized, then all of the drive and load constraints are separated out.  Solvit article Methodology-47 contains a script, which characterizes each sub-block at the top of the hierarchy and separates out the drive and load constraints via a Perl script.

# Example Scripts

## Synthesis experimentation script

This script shows how a block could be compiled with different strategies.  It is not necessary to compile a design with each strategy; a designer may want to immediately move to using a critical range, boundary optimization and DC-Ultra.  However an area constraint may add significant run-time, therefore the designer may wish to try both strategies and weigh the tradeoffs.

sh cat synthesis_experimentation.scr

/*

Title: synthesis_experimentation.scr

Last Update: 9 July 1998

Author: JJ

Description: This script shows examples of several basic compile

strategies to try.  The best timing will probably be

seen by using a critical range, area constraint,

boundary optimization, and DC-Ultra mode.  The critical

range and area constraint will increase run-time.

The BASENAME variable is used to control the name of

files that are written out.  The idea is to write out

.db’s and reports using the basename plus the REV

variable allowing the user to later track the files.

*/

/* Constants */

QUIT_AT_END       = true

MAIL_AT_END       = true

READ_ELAB         = true

REV               = 1

BASENAME          = "testcase"

ELAB_DB_TO_READ   = "testcase_elab.db"

MAPPED_DB_TO_READ = "testcase_gates.db"

/* Read either the elaborated .db or a previously mapped .db */

if (READ_ELAB == "true") {

read ELAB_DB_TO_READ

} else {

read MAPPED_DB_TO_READ

}

/* Compile the design */

include start_time.scr -quiet

/* REV1 performs a basic top-down compile */

if (REV == 1) {

remove_attribute current_design max_area

set_critical_range 0.0 current_design

compile -map_effort medium

/* REV2 adds boundary optimization */

} else if (REV == 2) {

remove_attribute current_design max_area

set_critical_range 0.0 current_design

compile -map_effort medium -boundary_opt

/* REV3 adds a critical range */

} else if (REV == 3) {

remove_attribute current_design max_area

set_critical_range 0.20 current_design

compile -map_effort medium -boundary_opt

/* REV4 adds a max_area constraint */

} else if (REV == 4) {

set_max_area 10000

set_critical_range 0.20 current_design

compile -map_effort medium -boundary_opt

/* REV5 adds SOP flattening of blockA and blockB */

} else if (REV == 5) {

set_max_area 10000

set_critical_range 0.20 current_design

set_flatten -phase true -design {blockA blockB} -effort medium

compile -map_effort medium -boundary_opt

/* REV10 does a DC-Ultra high effort incremental compile */

} else if (REV == 10) {

set_critical_range 0.15 current_design

set_ultra_mode -force

compile -map_effort high -boundary_opt -inc

}

include end_time.scr -quiet

write -hier -out BASENAME + "_" + REV + ".db"

/* Generate metrics */

report_constraint

report_area

report_timing -net -trans -sig 3

report_timing -path end -max_paths 50

report_timing -net -trans -sig 3 -max_paths 100 > somefile

include find_dw_implementations.scr -quiet

/* Send email to user indicating compile is finished */

if (EMAIL_AT_END == "true") {

sh cat command.log | /usr/ucb/mail -s "Compile_Finished" $USER

}

if (QUIT_AT_END == true) {

quit

}